Main + components source code bundle

main/app_flow.c

#include "app_flow.h"

#include "login_app.h"

#include "quiz_app.h"

static void app_flow_on_login(bool success, const char *token, const char *user)

{

(void)token;

if (!success)

{

return;

}

quiz_app_set_user_id(user);

login_app_destroy();

quiz_app_create_ui();

}

void app_flow_start(void)

{

login_app_set_result_cb(app_flow_on_login);

login_app_show();

}

main/app_flow.h

#ifndef APP_FLOW_H

#define APP_FLOW_H

#ifdef __cplusplus

extern "C" {

#endif

void app_flow_start(void);

#ifdef __cplusplus

}

#endif

#endif

main/app_network.c

#include "app_network.h"

#include <string.h>

#include "esp_event.h"

#include "esp_log.h"

#include "esp_netif.h"

#include "esp_wifi.h"

#include "nvs_flash.h"

#include "freertos/event_groups.h"

#include "user_config.h"

#define WIFI_CONNECTED_BIT BIT0

static const char *TAG = "app_network";

static EventGroupHandle_t s_wifi_event_group;

static bool s_wifi_started;

static void app_network_init_netif(void)

{

static bool netif_ready = false;

static bool event_loop_ready = false;

if (!netif_ready)

{

esp_err_t ret = esp_netif_init();

if (ret == ESP_OK || ret == ESP_ERR_INVALID_STATE)

{

netif_ready = true;

}

else

{

ESP_ERROR_CHECK(ret);

}

}

if (!event_loop_ready)

{

esp_err_t ret = esp_event_loop_create_default();

if (ret == ESP_OK || ret == ESP_ERR_INVALID_STATE)

{

event_loop_ready = true;

}

else

{

ESP_ERROR_CHECK(ret);

}

}

}

static void app_network_wifi_event_handler(void *arg, esp_event_base_t event_base, int32_t event_id, void *event_data)

{

(void)arg;

(void)event_data;

if (event_base == WIFI_EVENT && event_id == WIFI_EVENT_STA_START)

{

esp_wifi_connect();

}

else if (event_base == WIFI_EVENT && event_id == WIFI_EVENT_STA_DISCONNECTED)

{

esp_wifi_connect();

if (s_wifi_event_group)

{

xEventGroupClearBits(s_wifi_event_group, WIFI_CONNECTED_BIT);

}

ESP_LOGW(TAG, "Wi-Fi disconnected, reconnecting...");

}

else if (event_base == IP_EVENT && event_id == IP_EVENT_STA_GOT_IP)

{

if (s_wifi_event_group)

{

xEventGroupSetBits(s_wifi_event_group, WIFI_CONNECTED_BIT);

}

}

}

static void app_network_start_wifi(void)

{

if (s_wifi_started)

{

return;

}

if (!s_wifi_event_group)

{

s_wifi_event_group = xEventGroupCreate();

if (!s_wifi_event_group)

{

ESP_LOGE(TAG, "Failed to create Wi-Fi event group");

return;

}

}

esp_err_t nvs_ret = nvs_flash_init();

if (nvs_ret == ESP_ERR_NVS_NO_FREE_PAGES || nvs_ret == ESP_ERR_NVS_NEW_VERSION_FOUND)

{

ESP_ERROR_CHECK(nvs_flash_erase());

ESP_ERROR_CHECK(nvs_flash_init());

}

app_network_init_netif();

esp_netif_create_default_wifi_sta();

wifi_init_config_t cfg = WIFI_INIT_CONFIG_DEFAULT();

ESP_ERROR_CHECK(esp_wifi_init(&cfg));

ESP_ERROR_CHECK(esp_event_handler_register(WIFI_EVENT, ESP_EVENT_ANY_ID, &app_network_wifi_event_handler, NULL));

ESP_ERROR_CHECK(esp_event_handler_register(IP_EVENT, IP_EVENT_STA_GOT_IP, &app_network_wifi_event_handler, NULL));

wifi_config_t wifi_config = {0};

strlcpy((char *)wifi_config.sta.ssid, APP_WIFI_SSID, sizeof(wifi_config.sta.ssid));

strlcpy((char *)wifi_config.sta.password, APP_WIFI_PASS, sizeof(wifi_config.sta.password));

wifi_config.sta.threshold.authmode = WIFI_AUTH_WPA2_PSK;

ESP_ERROR_CHECK(esp_wifi_set_mode(WIFI_MODE_STA));

ESP_ERROR_CHECK(esp_wifi_set_config(WIFI_IF_STA, &wifi_config));

ESP_ERROR_CHECK(esp_wifi_start());

s_wifi_started = true;

ESP_LOGI(TAG, "Wi-Fi connecting to %s...", APP_WIFI_SSID);

}

bool app_network_wait_for_wifi(TickType_t timeout_ticks)

{

app_network_start_wifi();

if (!s_wifi_event_group)

{

ESP_LOGE(TAG, "Wi-Fi event group not ready");

return false;

}

EventBits_t bits = xEventGroupWaitBits(s_wifi_event_group, WIFI_CONNECTED_BIT, pdFALSE, pdTRUE, timeout_ticks);

return (bits & WIFI_CONNECTED_BIT) != 0;

}

main/app_network.h

#ifndef APP_NETWORK_H

#define APP_NETWORK_H

#include <stdbool.h>

#include "freertos/FreeRTOS.h"

#ifdef __cplusplus

extern "C" {

#endif

bool app_network_wait_for_wifi(TickType_t timeout_ticks);

#ifdef __cplusplus

}

#endif

#endif

main/login_app.c

#include "login_app.h"

#include <stdio.h>

#include <stdlib.h>

#include <string.h>

#include "esp_err.h"

#include "esp_http_client.h"

#include "esp_log.h"

#include "lvgl.h"

#include "app_network.h"

#include "user_config.h"

static const char *TAG = "login_app";

static lv_obj_t *s_login_screen;

static lv_obj_t *s_account_ta;

static lv_obj_t *s_password_ta;

static lv_obj_t *s_digit_roller;

static lv_obj_t *s_ok_btn;

static lv_obj_t *s_del_btn;

static lv_obj_t *s_signin_btn;

static lv_obj_t *s_status_label;

static bool s_request_inflight;

static login_app_result_cb s_result_cb;

typedef enum

{

INPUT_TARGET_ACCOUNT = 0,

INPUT_TARGET_PASSWORD,

} input_target_t;

static input_target_t s_active_target;

static char s_account_buf[64];

static char s_password_buf[64];

static int s_current_digit;

static void login_app_update_status(const char *text)

{

if (s_status_label && text)

{

lv_label_set_text(s_status_label, text);

}

}

static void login_app_set_loading(bool loading)

{

s_request_inflight = loading;

if (!s_signin_btn)

{

return;

}

if (loading)

{

lv_obj_add_state(s_signin_btn, LV_STATE_DISABLED);

}

else

{

lv_obj_clear_state(s_signin_btn, LV_STATE_DISABLED);

}

}

static void login_app_textarea_select_event(lv_event_t *e)

{

lv_obj_t *ta = lv_event_get_target(e);

if (ta == s_account_ta)

{

s_active_target = INPUT_TARGET_ACCOUNT;

lv_obj_add_state(s_account_ta, LV_STATE_FOCUSED);

lv_obj_clear_state(s_password_ta, LV_STATE_FOCUSED);

}

else if (ta == s_password_ta)

{

s_active_target = INPUT_TARGET_PASSWORD;

lv_obj_add_state(s_password_ta, LV_STATE_FOCUSED);

lv_obj_clear_state(s_account_ta, LV_STATE_FOCUSED);

}

}

static void login_app_roller_event(lv_event_t *e)

{

if (lv_event_get_code(e) != LV_EVENT_VALUE_CHANGED)

{

return;

}

char buf[4];

lv_roller_get_selected_str(lv_event_get_target(e), buf, sizeof(buf));

s_current_digit = atoi(buf);

}

static void login_app_roller_snap_event(lv_event_t *e)

{

if (lv_event_get_code(e) != LV_EVENT_SCROLL_END)

{

return;

}

lv_obj_t *roller = lv_event_get_target(e);

uint16_t selected = lv_roller_get_selected(roller);

lv_roller_set_selected(roller, selected, LV_ANIM_OFF);

}

static void login_app_ok_event(lv_event_t *e)

{

if (lv_event_get_code(e) != LV_EVENT_CLICKED)

{

return;

}

char digit_char = '0' + s_current_digit;

if (s_active_target == INPUT_TARGET_ACCOUNT)

{

size_t len = strlen(s_account_buf);

s_account_buf[len] = digit_char;

s_account_buf[len + 1] = '\0';

lv_textarea_set_text(s_account_ta, s_account_buf);

}

else

{

size_t len = strlen(s_password_buf);

s_password_buf[len] = digit_char;

s_password_buf[len + 1] = '\0';

lv_textarea_set_text(s_password_ta, s_password_buf);

}

}

static void login_app_del_event(lv_event_t *e)

{

if (lv_event_get_code(e) != LV_EVENT_CLICKED)

{

return;

}

char *buf = (s_active_target == INPUT_TARGET_ACCOUNT)

? s_account_buf

: s_password_buf;

lv_obj_t *ta = (s_active_target == INPUT_TARGET_ACCOUNT)

? s_account_ta

: s_password_ta;

size_t len = strlen(buf);

if (len == 0)

{

return;

}

buf[len - 1] = '\0';

lv_textarea_set_text(ta, buf);

}

static esp_err_t login_app_do_auth(const char *username,

const char *password,

int *out_status)

{

if (!username || !password || !out_status)

{

return ESP_ERR_INVALID_ARG;

}

*out_status = 0;

if (!app_network_wait_for_wifi(pdMS_TO_TICKS(10000)))

{

ESP_LOGE(TAG, "Wi-Fi not ready");

return ESP_ERR_TIMEOUT;

}

char payload[128];

snprintf(payload, sizeof(payload),

"{\"username\":\"%s\",\"password\":\"%s\"}",

username, password);

esp_http_client_config_t config = {

.host = APP_SERVER_HOST,

.port = APP_SERVER_PORT,

.path = APP_API_LOGIN_PATH,

.timeout_ms = 8000,

};

esp_http_client_handle_t client = esp_http_client_init(&config);

if (!client)

{

return ESP_FAIL;

}

esp_http_client_set_method(client, HTTP_METHOD_POST);

esp_http_client_set_header(client, "Content-Type", "application/json");

esp_http_client_set_post_field(client, payload, strlen(payload));

esp_err_t err = esp_http_client_perform(client);

if (err != ESP_OK)

{

ESP_LOGE(TAG, "HTTP perform failed: %s", esp_err_to_name(err));

esp_http_client_cleanup(client);

return err;

}

int status = esp_http_client_get_status_code(client);

*out_status = status;

ESP_LOGI(TAG, "HTTP status = %d", status);

esp_http_client_cleanup(client);

return ESP_OK;

}

static void login_app_handle_submit(lv_event_t *e)

{

(void)e;

if (s_request_inflight)

{

return;

}

const char *account = s_account_buf;

const char *password = s_password_buf;

if (!account || account[0] == '\0' || !password || password[0] == '\0')

{

login_app_update_status("Account or password empty");

return;

}

login_app_set_loading(true);

login_app_update_status("Signing in...");

int http_status = 0;

esp_err_t err = login_app_do_auth(account, password, &http_status);

login_app_set_loading(false);

if (err != ESP_OK)

{

login_app_update_status("Network request failed");

if (s_result_cb)

{

s_result_cb(false, NULL, NULL);

}

return;

}

if (http_status == 200)

{

login_app_update_status("Login successful");

if (s_result_cb)

{

s_result_cb(true, NULL, account);

}

}

else if (http_status == 401)

{

login_app_update_status("Invalid account or password");

s_account_buf[0] = '\0';

s_password_buf[0] = '\0';

if (s_account_ta)

{

lv_textarea_set_text(s_account_ta, "");

}

if (s_password_ta)

{

lv_textarea_set_text(s_password_ta, "");

}

if (s_result_cb)

{

s_result_cb(false, NULL, NULL);

}

}

else

{

login_app_update_status("Login failed");

if (s_result_cb)

{

s_result_cb(false, NULL, NULL);

}

}

}

static void login_app_build_ui(void)

{

s_login_screen = lv_obj_create(NULL);

lv_obj_set_style_bg_color(s_login_screen, lv_color_hex(0xf6f8fb), LV_PART_MAIN);

lv_obj_set_style_pad_all(s_login_screen, 4, LV_PART_MAIN);

lv_obj_t *row = lv_obj_create(s_login_screen);

lv_obj_set_size(row, lv_pct(100), lv_pct(100));

lv_obj_align(row, LV_ALIGN_CENTER, 0, 0);

lv_obj_set_style_pad_all(row, 4, LV_PART_MAIN);

lv_obj_set_flex_flow(row, LV_FLEX_FLOW_ROW);

lv_obj_set_flex_align(row,

LV_FLEX_ALIGN_SPACE_BETWEEN,

LV_FLEX_ALIGN_CENTER,

LV_FLEX_ALIGN_CENTER);

lv_obj_t *left = lv_obj_create(row);

lv_obj_set_flex_grow(left, 4);

lv_obj_set_flex_flow(left, LV_FLEX_FLOW_COLUMN);

lv_obj_set_style_bg_opa(left, LV_OPA_TRANSP, LV_PART_MAIN);

lv_obj_set_style_border_width(left, 0, LV_PART_MAIN);

lv_obj_set_style_bg_color(left, lv_color_hex(0xFFFFFF), LV_PART_MAIN);

lv_obj_set_style_bg_opa(left, LV_OPA_100, LV_PART_MAIN);

lv_obj_set_style_radius(left, 8, LV_PART_MAIN);

lv_obj_set_style_pad_all(left, 8, LV_PART_MAIN);

lv_obj_set_style_pad_gap(left, 10, LV_PART_MAIN);

s_account_ta = lv_textarea_create(left);

lv_textarea_set_one_line(s_account_ta, true);

lv_textarea_set_placeholder_text(s_account_ta, "Account");

lv_obj_set_height(s_account_ta, 48);

lv_obj_set_width(s_account_ta, lv_pct(92));

lv_obj_add_event_cb(s_account_ta, login_app_textarea_select_event, LV_EVENT_CLICKED, NULL);

s_password_ta = lv_textarea_create(left);

lv_textarea_set_one_line(s_password_ta, true);

lv_textarea_set_password_mode(s_password_ta, true);

lv_textarea_set_placeholder_text(s_password_ta, "Password");

lv_obj_set_height(s_password_ta, 48);

lv_obj_set_width(s_password_ta, lv_pct(92));

lv_obj_add_event_cb(s_password_ta, login_app_textarea_select_event, LV_EVENT_CLICKED, NULL);

s_active_target = INPUT_TARGET_ACCOUNT;

lv_obj_add_state(s_account_ta, LV_STATE_FOCUSED);

lv_obj_clear_state(s_password_ta, LV_STATE_FOCUSED);

lv_obj_t *mid = lv_obj_create(row);

lv_obj_set_flex_grow(mid, 2);

lv_obj_set_style_bg_opa(mid, LV_OPA_TRANSP, LV_PART_MAIN);

lv_obj_set_style_border_width(mid, 0, LV_PART_MAIN);

s_digit_roller = lv_roller_create(mid);

lv_obj_set_width(s_digit_roller, lv_pct(100));

lv_obj_set_height(s_digit_roller, lv_pct(80));

lv_obj_align(s_digit_roller, LV_ALIGN_CENTER, 0, 0);

lv_obj_set_style_bg_color(s_digit_roller, lv_color_hex(0xFFFFFF), LV_PART_MAIN);

lv_obj_set_style_bg_opa(s_digit_roller, LV_OPA_100, LV_PART_MAIN);

lv_obj_set_style_radius(s_digit_roller, 10, LV_PART_MAIN);

lv_obj_set_style_text_font(s_digit_roller, &lv_font_montserrat_32, LV_PART_MAIN);

lv_obj_set_style_text_color(s_digit_roller,

lv_palette_main(LV_PALETTE_GREY),

LV_PART_MAIN);

lv_obj_set_style_text_font(s_digit_roller, &lv_font_montserrat_48, LV_PART_SELECTED);

lv_obj_set_style_text_color(s_digit_roller,

lv_color_black(),

LV_PART_SELECTED);

lv_obj_set_style_bg_color(s_digit_roller,

lv_palette_lighten(LV_PALETTE_BLUE, 4),

LV_PART_SELECTED);

lv_obj_set_style_bg_opa(s_digit_roller,

LV_OPA_30,

LV_PART_SELECTED);

lv_obj_set_style_radius(s_digit_roller,

6,

LV_PART_SELECTED);

lv_obj_set_style_pad_top(s_digit_roller, 0, LV_PART_MAIN);

lv_obj_set_style_pad_bottom(s_digit_roller, 0, LV_PART_MAIN);

lv_roller_set_options(s_digit_roller,

"0\n1\n2\n3\n4\n5\n6\n7\n8\n9",

LV_ROLLER_MODE_INFINITE);

lv_roller_set_visible_row_count(s_digit_roller, 3);

lv_roller_set_selected(s_digit_roller, 0, LV_ANIM_OFF);

lv_obj_clear_flag(s_digit_roller, LV_OBJ_FLAG_SCROLL_ELASTIC);

lv_obj_add_event_cb(s_digit_roller,

login_app_roller_event,

LV_EVENT_VALUE_CHANGED,

NULL);

lv_obj_add_event_cb(s_digit_roller,

login_app_roller_snap_event,

LV_EVENT_SCROLL_END,

NULL);

s_current_digit = 0;

lv_obj_t *right = lv_obj_create(row);

lv_obj_set_flex_grow(right, 2);

lv_obj_set_height(right, lv_pct(100));

lv_obj_set_flex_flow(right, LV_FLEX_FLOW_COLUMN);

lv_obj_clear_flag(right, LV_OBJ_FLAG_SCROLLABLE);

lv_obj_set_style_bg_opa(right, LV_OPA_TRANSP, LV_PART_MAIN);

lv_obj_set_style_border_width(right, 0, LV_PART_MAIN);

lv_obj_set_style_pad_all(right, 0, LV_PART_MAIN);

lv_obj_set_flex_align(right,

LV_FLEX_ALIGN_SPACE_EVENLY,

LV_FLEX_ALIGN_CENTER,

LV_FLEX_ALIGN_CENTER);

s_ok_btn = lv_btn_create(right);

lv_obj_add_event_cb(s_ok_btn, login_app_ok_event, LV_EVENT_CLICKED, NULL);

lv_label_create(s_ok_btn);

lv_label_set_text(lv_obj_get_child(s_ok_btn, -1), "OK");

s_del_btn = lv_btn_create(right);

lv_obj_add_event_cb(s_del_btn, login_app_del_event, LV_EVENT_CLICKED, NULL);

lv_label_create(s_del_btn);

lv_label_set_text(lv_obj_get_child(s_del_btn, -1), "DEL");

s_signin_btn = lv_btn_create(right);

lv_obj_add_event_cb(s_signin_btn, login_app_handle_submit, LV_EVENT_CLICKED, NULL);

lv_label_create(s_signin_btn);

lv_label_set_text(lv_obj_get_child(s_signin_btn, -1), "Sign In");

const int BTN_H = 44;

lv_obj_set_width(s_ok_btn, lv_pct(90));

lv_obj_set_width(s_del_btn, lv_pct(90));

lv_obj_set_width(s_signin_btn, lv_pct(90));

lv_obj_set_height(s_ok_btn, BTN_H);

lv_obj_set_height(s_del_btn, BTN_H);

lv_obj_set_height(s_signin_btn, 52);

lv_obj_set_style_bg_color(s_signin_btn, lv_palette_main(LV_PALETTE_BLUE), LV_PART_MAIN);

lv_obj_set_style_text_color(s_signin_btn, lv_color_white(), LV_PART_MAIN);

lv_obj_set_style_radius(s_signin_btn, 10, LV_PART_MAIN);

lv_obj_set_style_radius(s_ok_btn, 10, LV_PART_MAIN);

lv_obj_set_style_radius(s_del_btn, 10, LV_PART_MAIN);

s_account_buf[0] = '\0';

s_password_buf[0] = '\0';

lv_textarea_set_text(s_account_ta, "");

lv_textarea_set_text(s_password_ta, "");

s_status_label = lv_label_create(s_login_screen);

lv_label_set_text(s_status_label, "Enter account and password");

lv_obj_set_style_text_color(s_status_label, lv_palette_main(LV_PALETTE_GREY), LV_PART_MAIN);

lv_obj_set_width(s_status_label, lv_pct(100));

lv_label_set_long_mode(s_status_label, LV_LABEL_LONG_WRAP);

lv_obj_align(s_status_label, LV_ALIGN_BOTTOM_MID, 0, -4);

}

void login_app_show(void)

{

if (!s_login_screen)

{

login_app_build_ui();

}

lv_scr_load(s_login_screen);

login_app_update_status("Enter account and password");

}

void login_app_destroy(void)

{

if (s_login_screen)

{

lv_obj_del_async(s_login_screen);

s_login_screen = NULL;

s_account_ta = NULL;

s_password_ta = NULL;

s_digit_roller = NULL;

s_ok_btn = NULL;

s_del_btn = NULL;

s_signin_btn = NULL;

s_status_label = NULL;

}

s_request_inflight = false;

s_account_buf[0] = '\0';

s_password_buf[0] = '\0';

}

void login_app_set_result_cb(login_app_result_cb cb)

{

s_result_cb = cb;

}

main/login_app.h

#ifndef LOGIN_APP_H

#define LOGIN_APP_H

#include <stdbool.h>

#ifdef __cplusplus

extern "C" {

#endif

typedef void (*login_app_result_cb)(bool success, const char *token, const char *user);

void login_app_show(void);

void login_app_destroy(void);

void login_app_set_result_cb(login_app_result_cb cb);

#ifdef __cplusplus

}

#endif

#endif

main/main.cpp

#include <stdio.h>

extern "C" {

#include "app_flow.h"

}

#include "freertos/FreeRTOS.h"

#include "freertos/task.h"

#include "freertos/event_groups.h"

#include "driver/spi_master.h"

#include "esp_timer.h"

#include "esp_lcd_panel_io.h"

#include "esp_lcd_panel_vendor.h"

#include "esp_lcd_panel_ops.h"

#include "esp_err.h"

#include "esp_log.h"

#include "esp_heap_caps.h"

#include "driver/gpio.h"

#include "lvgl.h"

#include "lv_demos.h"

#include "user_config.h"

#include "esp_lcd_axs15231b.h"

#include "i2c_bsp.h"

#include "button_bsp.h"

#include "esp_io_expander_tca9554.h"

#include "lcd_bl_pwm_bsp.h"

static const char *TAG = "example";

static SemaphoreHandle_t lvgl_mux = NULL;

static uint16_t *lvgl_dma_buf = NULL;

static SemaphoreHandle_t lvgl_flush_semap;

#if (Rotated == USER_DISP_ROT_90)

uint16_t* rotat_ptr = NULL;

#endif

static esp_io_expander_handle_t io_expander = NULL;

static bool is_vbatpowerflag = false;

#define LCD_BIT_PER_PIXEL (16)

static void example_lvgl_touch_cb(lv_indev_drv_t *drv, lv_indev_data_t *data);

static bool example_lvgl_lock(int timeout_ms);

static void example_lvgl_unlock(void);

static void example_lvgl_flush_cb(lv_disp_drv_t *drv, const lv_area_t *area, lv_color_t *color_map);

void example_lvgl_port_task(void *arg);

static void example_backlight_loop_task(void *arg);

static void power_Test(void *arg);

static void tca9554_init(void);

static void example_button_pwr_task(void *arg);

static const axs15231b_lcd_init_cmd_t lcd_init_cmds[] =

{

{0x11, (uint8_t []){0x00}, 0, 100},

{0x29, (uint8_t []){0x00}, 0, 100},

};

static bool example_notify_lvgl_flush_ready(esp_lcd_panel_io_handle_t panel_io, esp_lcd_panel_io_event_data_t *edata, void *user_ctx)

{

BaseType_t TaskWoken;

xSemaphoreGiveFromISR(lvgl_flush_semap, &TaskWoken);

return false;

}

static void example_increase_lvgl_tick(void *arg)

{

lv_tick_inc(EXAMPLE_LVGL_TICK_PERIOD_MS);

}

static void power_Test(void *arg)

{

if (gpio_get_level(GPIO_NUM_16))

{

is_vbatpowerflag = true;

}

vTaskDelete(NULL);

}

static void tca9554_init(void)

{

i2c_master_bus_handle_t tca9554_i2c_bus_ = NULL;

ESP_ERROR_CHECK(i2c_master_get_bus_handle(0, &tca9554_i2c_bus_));

esp_io_expander_new_i2c_tca9554(tca9554_i2c_bus_, ESP_IO_EXPANDER_I2C_TCA9554_ADDRESS_000, &io_expander);

esp_io_expander_set_dir(io_expander, IO_EXPANDER_PIN_NUM_6, IO_EXPANDER_OUTPUT);

esp_io_expander_set_level(io_expander, IO_EXPANDER_PIN_NUM_6, 1);

}

static void example_button_pwr_task(void* arg)

{

for (;;)

{

EventBits_t even = xEventGroupWaitBits(pwr_groups, set_bit_all, pdTRUE, pdFALSE, pdMS_TO_TICKS(2 * 1000));

if (get_bit_button(even, 0))

{

}

else if (get_bit_button(even, 1))

{

if (is_vbatpowerflag)

{

is_vbatpowerflag = false;

esp_io_expander_set_level(io_expander, IO_EXPANDER_PIN_NUM_6, 0);

}

}

else if (get_bit_button(even, 2))

{

if (!is_vbatpowerflag)

{

is_vbatpowerflag = true;

}

}

}

}

extern "C" void app_main(void)

{

lcd_bl_pwm_bsp_init(LCD_PWM_MODE_255);

#if (Rotated == USER_DISP_ROT_90)

rotat_ptr = (uint16_t*)heap_caps_malloc(EXAMPLE_LCD_H_RES * EXAMPLE_LCD_V_RES * sizeof(uint16_t), MALLOC_CAP_SPIRAM);

assert(rotat_ptr);

#endif

lvgl_flush_semap = xSemaphoreCreateBinary();

i2c_master_Init();

tca9554_init();

button_Init();

xTaskCreatePinnedToCore(example_button_pwr_task, "example_button_pwr_task", 4 * 1024, NULL, 2, NULL, 1);

xTaskCreatePinnedToCore(power_Test, "power_Test", 4 * 1024, NULL, 3, NULL, 1);

static lv_disp_draw_buf_t disp_buf;

static lv_disp_drv_t disp_drv;

ESP_LOGI(TAG, "Initialize LCD RESET GPIO");

gpio_config_t gpio_conf = {};

gpio_conf.intr_type = GPIO_INTR_DISABLE;

gpio_conf.mode = GPIO_MODE_OUTPUT;

gpio_conf.pin_bit_mask = ((uint64_t)0X01<<EXAMPLE_PIN_NUM_LCD_RST);

gpio_conf.pull_down_en = GPIO_PULLDOWN_DISABLE;

gpio_conf.pull_up_en = GPIO_PULLUP_ENABLE;

ESP_ERROR_CHECK_WITHOUT_ABORT(gpio_config(&gpio_conf));

ESP_LOGI(TAG, "Initialize QSPI bus");

spi_bus_config_t buscfg = {};

buscfg.data0_io_num = EXAMPLE_PIN_NUM_LCD_DATA0;

buscfg.data1_io_num = EXAMPLE_PIN_NUM_LCD_DATA1;

buscfg.sclk_io_num = EXAMPLE_PIN_NUM_LCD_PCLK;

buscfg.data2_io_num = EXAMPLE_PIN_NUM_LCD_DATA2;

buscfg.data3_io_num = EXAMPLE_PIN_NUM_LCD_DATA3;

buscfg.max_transfer_sz = LVGL_DMA_BUFF_LEN;

ESP_ERROR_CHECK(spi_bus_initialize(LCD_HOST, &buscfg, SPI_DMA_CH_AUTO));

ESP_LOGI(TAG, "Install panel IO");

esp_lcd_panel_io_handle_t panel_io = NULL;

esp_lcd_panel_handle_t panel = NULL;

esp_lcd_panel_io_spi_config_t io_config = {};

io_config.cs_gpio_num = EXAMPLE_PIN_NUM_LCD_CS;

io_config.dc_gpio_num = -1;

io_config.spi_mode = 3;

io_config.pclk_hz = 40 * 1000 * 1000;

io_config.trans_queue_depth = 10;

io_config.on_color_trans_done = example_notify_lvgl_flush_ready;

io_config.lcd_cmd_bits = 32;

io_config.lcd_param_bits = 8;

io_config.flags.quad_mode = true;

ESP_ERROR_CHECK(esp_lcd_new_panel_io_spi(LCD_HOST, &io_config, &panel_io));

axs15231b_vendor_config_t vendor_config = {};

vendor_config.flags.use_qspi_interface = 1;

vendor_config.init_cmds = lcd_init_cmds;

vendor_config.init_cmds_size = sizeof(lcd_init_cmds) / sizeof(lcd_init_cmds[0]);

esp_lcd_panel_dev_config_t panel_config = {};

panel_config.reset_gpio_num = -1;

panel_config.rgb_ele_order = LCD_RGB_ELEMENT_ORDER_RGB;

panel_config.bits_per_pixel = LCD_BIT_PER_PIXEL;

panel_config.vendor_config = &vendor_config;

ESP_LOGI(TAG, "Install panel driver");

ESP_ERROR_CHECK(esp_lcd_new_panel_axs15231b(panel_io, &panel_config, &panel));

ESP_ERROR_CHECK(gpio_set_level(EXAMPLE_PIN_NUM_LCD_RST, 1));

vTaskDelay(pdMS_TO_TICKS(30));

ESP_ERROR_CHECK(gpio_set_level(EXAMPLE_PIN_NUM_LCD_RST, 0));

vTaskDelay(pdMS_TO_TICKS(250));

ESP_ERROR_CHECK(gpio_set_level(EXAMPLE_PIN_NUM_LCD_RST, 1));

vTaskDelay(pdMS_TO_TICKS(30));

ESP_ERROR_CHECK(esp_lcd_panel_init(panel));

lv_init();

lvgl_dma_buf = (uint16_t *)heap_caps_malloc(LVGL_DMA_BUFF_LEN, MALLOC_CAP_DMA);

assert(lvgl_dma_buf);

lv_color_t *buffer_1 = (lv_color_t *)heap_caps_malloc(LVGL_SPIRAM_BUFF_LEN, MALLOC_CAP_SPIRAM);

lv_color_t *buffer_2 = (lv_color_t *)heap_caps_malloc(LVGL_SPIRAM_BUFF_LEN, MALLOC_CAP_SPIRAM);

assert(buffer_1);

assert(buffer_2);

lv_disp_draw_buf_init(&disp_buf, buffer_1, buffer_2, EXAMPLE_LCD_H_RES * EXAMPLE_LCD_V_RES);

ESP_LOGI(TAG, "Register display driver to LVGL");

lv_disp_drv_init(&disp_drv);

disp_drv.hor_res = EXAMPLE_LCD_H_RES;

disp_drv.ver_res = EXAMPLE_LCD_V_RES;

disp_drv.flush_cb = example_lvgl_flush_cb;

disp_drv.draw_buf = &disp_buf;

disp_drv.full_refresh = 1;

disp_drv.user_data = panel;

lv_disp_drv_register(&disp_drv);

ESP_LOGI(TAG, "Install LVGL tick timer");

esp_timer_create_args_t lvgl_tick_timer_args = {};

lvgl_tick_timer_args.callback = &example_increase_lvgl_tick;

lvgl_tick_timer_args.name = "lvgl_tick";

esp_timer_handle_t lvgl_tick_timer = NULL;

ESP_ERROR_CHECK(esp_timer_create(&lvgl_tick_timer_args, &lvgl_tick_timer));

ESP_ERROR_CHECK(esp_timer_start_periodic(lvgl_tick_timer, EXAMPLE_LVGL_TICK_PERIOD_MS * 1000));

static lv_indev_drv_t indev_drv;

lv_indev_drv_init(&indev_drv);

indev_drv.type = LV_INDEV_TYPE_POINTER;

indev_drv.read_cb = example_lvgl_touch_cb;

lv_indev_drv_register(&indev_drv);

lvgl_mux = xSemaphoreCreateMutex();

assert(lvgl_mux);

xTaskCreatePinnedToCore(example_lvgl_port_task, "LVGL", 4000, NULL, 4, NULL, 0);

xTaskCreatePinnedToCore(example_backlight_loop_task, "example_backlight_loop_task", 4 * 1024, NULL, 2, NULL, 0);

if (example_lvgl_lock(-1))

{

app_flow_start();

example_lvgl_unlock();

}

}

static void example_lvgl_flush_cb(lv_disp_drv_t *drv, const lv_area_t *area, lv_color_t *color_map)

{

#if (Rotated == USER_DISP_ROT_90)

uint32_t index = 0;

uint16_t *data_ptr = (uint16_t *)color_map;

for (uint16_t j = 0; j < EXAMPLE_LCD_H_RES; j++)

{

for (uint16_t i = 0; i < EXAMPLE_LCD_V_RES; i++)

{

rotat_ptr[index++] = data_ptr[EXAMPLE_LCD_H_RES * (EXAMPLE_LCD_V_RES - i - 1) + j];

}

}

#endif

esp_lcd_panel_handle_t panel_handle = (esp_lcd_panel_handle_t) drv->user_data;

const int flush_coun = (LVGL_SPIRAM_BUFF_LEN / LVGL_DMA_BUFF_LEN);

const int offgap = (LCD_NOROT_VRES / flush_coun);

const int dmalen = (LVGL_DMA_BUFF_LEN / 2);

int offsetx1 = 0;

int offsety1 = 0;

int offsetx2 = LCD_NOROT_HRES;

int offsety2 = offgap;

#if (Rotated == USER_DISP_ROT_90)

uint16_t *map = (uint16_t *)rotat_ptr;

#else

uint16_t *map = (uint16_t *)color_map;

#endif

xSemaphoreGive(lvgl_flush_semap);

for(int i = 0; i < flush_coun; i++)

{

xSemaphoreTake(lvgl_flush_semap, portMAX_DELAY);

memcpy(lvgl_dma_buf, map, LVGL_DMA_BUFF_LEN);

esp_lcd_panel_draw_bitmap(panel_handle, offsetx1, offsety1, offsetx2, offsety2, lvgl_dma_buf);

offsety1 += offgap;

offsety2 += offgap;

map += dmalen;

}

xSemaphoreTake(lvgl_flush_semap, portMAX_DELAY);

lv_disp_flush_ready(drv);

}

static bool example_lvgl_lock(int timeout_ms)

{

const TickType_t timeout_ticks = (timeout_ms == -1) ? portMAX_DELAY : pdMS_TO_TICKS(timeout_ms);

return xSemaphoreTake(lvgl_mux, timeout_ticks) == pdTRUE;

}

static void example_lvgl_unlock(void)

{

assert(lvgl_mux && "bsp_display_start must be called first");

xSemaphoreGive(lvgl_mux);

}

void example_lvgl_port_task(void *arg)

{

uint32_t task_delay_ms = EXAMPLE_LVGL_TASK_MAX_DELAY_MS;

for(;;)

{

if (example_lvgl_lock(-1))

{

task_delay_ms = lv_timer_handler();

example_lvgl_unlock();

}

if (task_delay_ms > EXAMPLE_LVGL_TASK_MAX_DELAY_MS)

{

task_delay_ms = EXAMPLE_LVGL_TASK_MAX_DELAY_MS;

}

else if (task_delay_ms < EXAMPLE_LVGL_TASK_MIN_DELAY_MS)

{

task_delay_ms = EXAMPLE_LVGL_TASK_MIN_DELAY_MS;

}

vTaskDelay(pdMS_TO_TICKS(task_delay_ms));

}

}

static void example_lvgl_touch_cb(lv_indev_drv_t *drv, lv_indev_data_t *data)

{

uint8_t read_touchpad_cmd[11] = {0xb5, 0xab, 0xa5, 0x5a, 0x0, 0x0, 0x0, 0x0e, 0x0, 0x0, 0x0};

uint8_t buff[32] = {0};

memset(buff, 0, 32);

esp_err_t touch_err = i2c_master_write_read_dev(disp_touch_dev_handle, read_touchpad_cmd, 11, buff, 32);

if (touch_err != ESP_OK)

{

data->state = LV_INDEV_STATE_REL;

return;

}

uint16_t pointX;

uint16_t pointY;

pointX = (((uint16_t)buff[2] & 0x0f) << 8) | (uint16_t)buff[3];

pointY = (((uint16_t)buff[4] & 0x0f) << 8) | (uint16_t)buff[5];

if (buff[1] > 0 && buff[1] < 5)

{

data->state = LV_INDEV_STATE_PR;

#if (Rotated == USER_DISP_ROT_NONO)

if (pointX > EXAMPLE_LCD_V_RES) pointX = EXAMPLE_LCD_V_RES;

if (pointY > EXAMPLE_LCD_H_RES) pointY = EXAMPLE_LCD_H_RES;

data->point.x = pointY;

data->point.y = (EXAMPLE_LCD_V_RES - pointX);

#else

if (pointX > EXAMPLE_LCD_H_RES) pointX = EXAMPLE_LCD_H_RES;

if (pointY > EXAMPLE_LCD_V_RES) pointY = EXAMPLE_LCD_V_RES;

data->point.x = (EXAMPLE_LCD_H_RES - pointX);

data->point.y = (EXAMPLE_LCD_V_RES - pointY);

#endif

}

else

{

data->state = LV_INDEV_STATE_REL;

}

}

static void example_backlight_loop_task(void *arg)

{

for(;;)

{

#if (Backlight_Testing == true)

vTaskDelay(pdMS_TO_TICKS(1500));

setUpduty(LCD_PWM_MODE_255);

vTaskDelay(pdMS_TO_TICKS(1500));

setUpduty(LCD_PWM_MODE_175);

vTaskDelay(pdMS_TO_TICKS(1500));

setUpduty(LCD_PWM_MODE_125);

vTaskDelay(pdMS_TO_TICKS(1500));

setUpduty(LCD_PWM_MODE_0);

#else

vTaskDelay(pdMS_TO_TICKS(2000));

#endif

}

}

main/quiz_app.c

#include "quiz_app.h"

#include <stdbool.h>

#include <stdint.h>

#include <stdio.h>

#include <stdlib.h>

#include <string.h>

#include "cJSON.h"

#include "esp_err.h"

#include "esp_log.h"

#include "esp_http_client.h"

#include "lvgl.h"

#include "freertos/FreeRTOS.h"

#include "freertos/task.h"

#include "app_network.h"

#include "user_config.h"

#define UI_FONT_LARGE   lv_theme_get_font_large(NULL)

#define UI_FONT_NORMAL  lv_theme_get_font_normal(NULL)

#define UI_FONT_SMALL   lv_theme_get_font_small(NULL)

#define OPTION_TEXT_NORMAL_COLOR   lv_color_hex(0x333333)

#define OPTION_TEXT_ACTIVE_COLOR   lv_palette_main(LV_PALETTE_BLUE)

#define QUIZ_MAX_QUESTIONS     12

#define QUIZ_OPTION_COUNT      4

#define QUIZ_TEXT_LEN          192

#define QUIZ_OPTION_LEN        96

typedef struct

{

char id[24];

int correct;

int your;

} quiz_wrong_item_t;

typedef struct

{

char id[24];

char stem[QUIZ_TEXT_LEN];

char options[QUIZ_OPTION_COUNT][QUIZ_OPTION_LEN];

uint8_t correct_index;

} quiz_question_t;

typedef struct

{

quiz_question_t questions[QUIZ_MAX_QUESTIONS];

uint8_t answers[QUIZ_MAX_QUESTIONS];

uint8_t question_count;

uint8_t current_question;

bool questions_ready;

bool test_finished;

int server_score;

int server_total;

uint8_t server_wrong_count;

quiz_wrong_item_t server_wrong[QUIZ_MAX_QUESTIONS];

} quiz_state_t;

static const char *TAG = "quiz_app";

static quiz_state_t s_state;

static char s_user_id[16];

static char s_quiz_id[40];

static lv_obj_t *s_home_screen;

static lv_obj_t *s_result_screen;

static lv_obj_t *s_test_screen;

static lv_obj_t *s_scroll;

static lv_obj_t *s_result_scroll;

static lv_obj_t *s_progress_label;

static lv_obj_t *s_question_label;

static lv_obj_t *s_option_labels[QUIZ_OPTION_COUNT];

static lv_obj_t *s_bottom_bar;

static lv_obj_t *s_option_btns[QUIZ_OPTION_COUNT];

static lv_obj_t *s_submit_btn;

static lv_obj_t *s_toast_label;

static lv_timer_t *s_toast_timer;

static void quiz_handle_download(lv_event_t *e);

static void quiz_handle_start_test(lv_event_t *e);

static void quiz_handle_view_results(lv_event_t *e);

static void quiz_handle_option(lv_event_t *e);

static void quiz_handle_submit(lv_event_t *e);

static void quiz_hide_toast_cb(lv_timer_t *t);

static void quiz_back_to_home(lv_event_t *e);

static esp_err_t quiz_http_get_questions(char **out_json);

static esp_err_t quiz_http_post_results(const char *payload);

static void quiz_load_question(uint8_t index);

static void quiz_build_result_screen(void);

static void quiz_show_results_screen(void);

static void quiz_finish_and_upload(void);

static void quiz_reset_server_result(void);

static const quiz_question_t *quiz_find_question_by_id(const char *id);

static void quiz_show_toast(const char *text, uint32_t duration_ms)

{

if (!text || text[0] == '\0')

{

return;

}

if (!s_toast_label)

{

s_toast_label = lv_label_create(lv_layer_top());

lv_obj_set_style_bg_opa(s_toast_label, LV_OPA_70, LV_PART_MAIN);

lv_obj_set_style_bg_color(s_toast_label, lv_color_black(), LV_PART_MAIN);

lv_obj_set_style_text_color(s_toast_label, lv_color_white(), LV_PART_MAIN);

lv_obj_set_style_text_font(s_toast_label, UI_FONT_NORMAL, LV_PART_MAIN);

lv_obj_set_style_pad_all(s_toast_label, 10, LV_PART_MAIN);

lv_obj_set_style_radius(s_toast_label, 10, LV_PART_MAIN);

}

lv_label_set_text(s_toast_label, text);

lv_obj_center(s_toast_label);

lv_obj_clear_flag(s_toast_label, LV_OBJ_FLAG_HIDDEN);

if (!s_toast_timer)

{

s_toast_timer = lv_timer_create(quiz_hide_toast_cb, duration_ms, NULL);

}

else

{

lv_timer_set_period(s_toast_timer, duration_ms);

lv_timer_reset(s_toast_timer);

}

lv_timer_set_repeat_count(s_toast_timer, 1);

}

static esp_err_t quiz_http_get_questions(char **out_json)

{

if (!out_json)

{

return ESP_ERR_INVALID_ARG;

}

if (s_user_id[0] == '\0')

{

ESP_LOGE(TAG, "User ID missing, cannot download questions");

return ESP_ERR_INVALID_STATE;

}

if (!app_network_wait_for_wifi(pdMS_TO_TICKS(10000)))

{

return ESP_ERR_TIMEOUT;

}

*out_json = NULL;

char url[128];

snprintf(url, sizeof(url),

"http://%s:%d/questions?user_id=%s",

APP_SERVER_HOST,

APP_SERVER_PORT,

s_user_id);

esp_http_client_config_t config = {

.url = url,

.method = HTTP_METHOD_GET,

.timeout_ms = 8000,

.disable_auto_redirect = false,

};

esp_http_client_handle_t client = esp_http_client_init(&config);

if (!client)

{

ESP_LOGE(TAG, "Failed to init client");

return ESP_FAIL;

}

esp_err_t err = esp_http_client_open(client, 0);

if (err != ESP_OK)

{

ESP_LOGE(TAG, "HTTP open failed: %s", esp_err_to_name(err));

esp_http_client_cleanup(client);

return err;

}

int content_length = esp_http_client_fetch_headers(client);

int status = esp_http_client_get_status_code(client);

if (status != 200)

{

ESP_LOGE(TAG, "Questions request failed, status=%d", status);

esp_http_client_close(client);

esp_http_client_cleanup(client);

return ESP_FAIL;

}

if (content_length <= 0)

{

ESP_LOGE(TAG, "Invalid content length: %d", content_length);

esp_http_client_close(client);

esp_http_client_cleanup(client);

return ESP_FAIL;

}

ESP_LOGI(TAG, "HTTP Content-Length = %d", content_length);

char *buffer = malloc(content_length + 1);

if (!buffer)

{

ESP_LOGE(TAG, "malloc failed");

esp_http_client_cleanup(client);

return ESP_ERR_NO_MEM;

}

int total_read = 0;

while (total_read < content_length)

{

int read_len = esp_http_client_read(client, buffer + total_read,

content_length - total_read);

if (read_len < 0)

{

ESP_LOGE(TAG, "Read error");

free(buffer);

esp_http_client_cleanup(client);

return ESP_FAIL;

}

if (read_len == 0)

{

break;

}

total_read += read_len;

}

buffer[total_read] = '\0';

ESP_LOGW(TAG, "HTTP BODY RECEIVED: %s", buffer);

*out_json = buffer;

esp_http_client_close(client);

esp_http_client_cleanup(client);

return ESP_OK;

}

static esp_err_t quiz_http_post_results(const char *payload)

{

if (!payload)

{

return ESP_ERR_INVALID_ARG;

}

if (!app_network_wait_for_wifi(pdMS_TO_TICKS(10000)))

{

return ESP_ERR_TIMEOUT;

}

esp_http_client_config_t config = {

.host = APP_SERVER_HOST,

.port = APP_SERVER_PORT,

.path = APP_API_SUBMIT_PATH,

.transport_type = HTTP_TRANSPORT_OVER_TCP,

.timeout_ms = 12000,

};

esp_http_client_handle_t client = esp_http_client_init(&config);

if (!client)

{

return ESP_FAIL;

}

esp_http_client_set_method(client, HTTP_METHOD_POST);

esp_http_client_set_header(client, "Content-Type", "application/json");

esp_http_client_set_post_field(client, payload, strlen(payload));

esp_err_t err = esp_http_client_perform(client);

if (err != ESP_OK)

{

esp_http_client_cleanup(client);

return err;

}

int status = esp_http_client_get_status_code(client);

if (status != 200)

{

esp_http_client_cleanup(client);

return ESP_FAIL;

}

esp_http_client_cleanup(client);

return ESP_OK;

}

static bool quiz_parse_questions(const char *json)

{

if (!json)

{

return false;

}

cJSON *root = cJSON_Parse(json);

if (!root)

{

ESP_LOGE(TAG, "Failed to parse JSON");

return false;

}

s_quiz_id[0] = '\0';

cJSON *quiz_id_js = cJSON_GetObjectItem(root, "quiz_id");

if (cJSON_IsString(quiz_id_js))

{

strncpy(s_quiz_id, quiz_id_js->valuestring, sizeof(s_quiz_id) - 1);

s_quiz_id[sizeof(s_quiz_id) - 1] = '\0';

}

cJSON *questions = cJSON_GetObjectItemCaseSensitive(root, "questions");

if (!cJSON_IsArray(questions))

{

cJSON_Delete(root);

ESP_LOGE(TAG, "Questions field is missing");

return false;

}

memset(&s_state, 0, sizeof(s_state));

s_state.question_count = 0;

cJSON *item = NULL;

cJSON_ArrayForEach(item, questions)

{

if (s_state.question_count >= QUIZ_MAX_QUESTIONS)

{

break;

}

cJSON *id = cJSON_GetObjectItemCaseSensitive(item, "id");

cJSON *stem = cJSON_GetObjectItemCaseSensitive(item, "stem");

cJSON *options = cJSON_GetObjectItemCaseSensitive(item, "options");

cJSON *answer = cJSON_GetObjectItemCaseSensitive(item, "answer");

if (!cJSON_IsArray(options) || cJSON_GetArraySize(options) < QUIZ_OPTION_COUNT)

{

ESP_LOGW(TAG, "Skip malformed question");

continue;

}

quiz_question_t *q = &s_state.questions[s_state.question_count];

strncpy(q->id, cJSON_IsString(id) ? id->valuestring : "NA", sizeof(q->id) - 1);

strncpy(q->stem, cJSON_IsString(stem) ? stem->valuestring : "Untitled question", sizeof(q->stem) - 1);

for (int i = 0; i < QUIZ_OPTION_COUNT; i++)

{

cJSON *opt = cJSON_GetArrayItem(options, i);

strncpy(q->options[i], cJSON_IsString(opt) ? opt->valuestring : "N/A", sizeof(q->options[i]) - 1);

}

q->correct_index = (uint8_t)(cJSON_IsNumber(answer) ? answer->valueint : 0);

if (q->correct_index >= QUIZ_OPTION_COUNT)

{

q->correct_index = 0;

}

s_state.question_count++;

}

cJSON_Delete(root);

s_state.questions_ready = (s_state.question_count > 0);

s_state.test_finished = false;

s_state.current_question = 0;

memset(s_state.answers, 0xFF, sizeof(s_state.answers));

ESP_LOGI(TAG, "Loaded %d questions", s_state.question_count);

return s_state.questions_ready;

}

static bool quiz_download_questions(void)

{

char *json = NULL;

esp_err_t err = quiz_http_get_questions(&json);

if (err != ESP_OK || !json)

{

quiz_show_toast("Download failed", 1800);

return false;

}

ESP_LOGW(TAG, "RAW JSON RECEIVED: %s", json);

bool ok = quiz_parse_questions(json);

free(json);

if (!ok)

{

ESP_LOGE(TAG, "Parse JSON failed, abort UI");

quiz_show_toast("Invalid data", 1800);

return false;

}

return true;

}

static void quiz_handle_download(lv_event_t *e)

{

LV_UNUSED(e);

if (!quiz_download_questions())

{

ESP_LOGE(TAG, "Download failed, not entering question screen");

return;

}

}

static void quiz_handle_start_test(lv_event_t *e)

{

LV_UNUSED(e);

if (!s_state.questions_ready)

{

quiz_show_toast("Please download questions first", 2000);

return;

}

s_state.current_question = 0;

s_state.test_finished = false;

quiz_reset_server_result();

memset(s_state.answers, 0xFF, sizeof(s_state.answers));

lv_scr_load(s_test_screen);

quiz_load_question(s_state.current_question);

}

static void quiz_handle_view_results(lv_event_t *e)

{

LV_UNUSED(e);

if (!s_state.test_finished)

{

quiz_show_toast("No test taken yet", 2000);

return;

}

quiz_show_results_screen();

}

static void quiz_create_home_screen(void)

{

s_home_screen = lv_obj_create(NULL);

lv_obj_set_style_bg_color(s_home_screen, lv_color_white(), LV_PART_MAIN);

lv_obj_set_style_text_color(s_home_screen, lv_color_hex(0x111111), LV_PART_MAIN);

lv_obj_set_style_pad_all(s_home_screen, 12, LV_PART_MAIN);

lv_obj_t *row = lv_obj_create(s_home_screen);

lv_obj_set_size(row, lv_pct(100), 100);

lv_obj_align(row, LV_ALIGN_CENTER, 0, -10);

lv_obj_set_flex_flow(row, LV_FLEX_FLOW_ROW);

lv_obj_set_flex_align(row, LV_FLEX_ALIGN_SPACE_EVENLY, LV_FLEX_ALIGN_CENTER, LV_FLEX_ALIGN_CENTER);

lv_obj_set_style_bg_opa(row, LV_OPA_0, LV_PART_MAIN);

lv_obj_set_style_pad_all(row, 10, LV_PART_MAIN);

static const char *btn_texts[3] = {"Download", "Start Test", "View Results"};

lv_event_cb_t handlers[3] = {quiz_handle_download, quiz_handle_start_test, quiz_handle_view_results};

for (int i = 0; i < 3; i++)

{

lv_obj_t *btn = lv_btn_create(row);

lv_obj_set_size(btn, 120, 70);

lv_obj_add_event_cb(btn, handlers[i], LV_EVENT_CLICKED, NULL);

lv_obj_set_style_bg_color(btn, lv_palette_main(LV_PALETTE_BLUE), LV_PART_MAIN);

lv_obj_set_style_radius(btn, 10, LV_PART_MAIN);

lv_obj_set_style_border_width(btn, 1, LV_PART_MAIN);

lv_obj_set_style_border_color(btn, lv_color_hex(0x2f6ee2), LV_PART_MAIN);

lv_obj_t *lbl = lv_label_create(btn);

lv_label_set_text(lbl, btn_texts[i]);

lv_obj_set_style_text_font(lbl, UI_FONT_NORMAL, LV_PART_MAIN);

lv_obj_center(lbl);

}

}

static void quiz_build_test_screen(void)

{

s_test_screen = lv_obj_create(NULL);

lv_obj_set_size(s_test_screen, 640, 172);

lv_obj_clear_flag(s_test_screen, LV_OBJ_FLAG_SCROLLABLE);

lv_obj_set_style_bg_color(s_test_screen, lv_color_white(), 0);

s_scroll = lv_obj_create(s_test_screen);

lv_obj_set_size(s_scroll, 640, 172);

lv_obj_align(s_scroll, LV_ALIGN_TOP_MID, 0, 0);

lv_obj_set_scroll_dir(s_scroll, LV_DIR_VER);

lv_obj_set_scrollbar_mode(s_scroll, LV_SCROLLBAR_MODE_AUTO);

lv_obj_clear_flag(s_scroll, LV_OBJ_FLAG_SCROLL_ELASTIC);

lv_obj_set_flex_flow(s_scroll, LV_FLEX_FLOW_COLUMN);

lv_obj_set_flex_align(s_scroll,

LV_FLEX_ALIGN_START,

LV_FLEX_ALIGN_START,

LV_FLEX_ALIGN_START);

lv_obj_set_style_pad_all(s_scroll, 0, 0);

lv_obj_set_style_pad_gap(s_scroll, 8, 0);

lv_obj_set_style_bg_opa(s_scroll, LV_OPA_TRANSP, 0);

lv_obj_set_style_border_width(s_scroll, 0, 0);

lv_obj_t *q_row = lv_obj_create(s_scroll);

lv_obj_set_width(q_row, LV_PCT(100));

lv_obj_set_height(q_row, LV_SIZE_CONTENT);

lv_obj_set_flex_flow(q_row, LV_FLEX_FLOW_ROW);

lv_obj_set_flex_align(q_row,

LV_FLEX_ALIGN_START,

LV_FLEX_ALIGN_START,

LV_FLEX_ALIGN_CENTER);

lv_obj_set_style_bg_opa(q_row, LV_OPA_TRANSP, 0);

lv_obj_set_style_border_width(q_row, 0, 0);

lv_obj_set_style_pad_left(q_row, 8, 0);

lv_obj_set_style_pad_right(q_row, 8, 0);

lv_obj_set_style_pad_top(q_row, 8, 0);

lv_obj_set_style_pad_bottom(q_row, 2, 0);

lv_obj_set_style_pad_gap(q_row, 8, 0);

lv_obj_set_style_min_height(q_row, 0, 0);

lv_obj_clear_flag(q_row, LV_OBJ_FLAG_SCROLLABLE);

s_question_label = lv_label_create(q_row);

lv_obj_set_flex_grow(s_question_label, 1);

lv_obj_set_width(s_question_label, LV_PCT(100));

lv_label_set_long_mode(s_question_label, LV_LABEL_LONG_WRAP);

lv_label_set_text(s_question_label, "Loading...");

lv_obj_set_style_text_font(s_question_label, UI_FONT_LARGE, 0);

s_progress_label = lv_label_create(q_row);

lv_label_set_text(s_progress_label, "0 / 0");

lv_obj_set_style_text_font(s_progress_label, UI_FONT_SMALL, 0);

lv_obj_set_style_text_align(s_progress_label, LV_TEXT_ALIGN_RIGHT, 0);

for (int i = 0; i < QUIZ_OPTION_COUNT; i++)

{

s_option_labels[i] = lv_label_create(s_scroll);

lv_obj_set_width(s_option_labels[i], LV_PCT(100));

lv_label_set_long_mode(s_option_labels[i], LV_LABEL_LONG_WRAP);

lv_label_set_text_fmt(s_option_labels[i], "%c. ", 'A' + i);

lv_obj_set_style_pad_left(s_option_labels[i], 10, 0);

lv_obj_set_style_pad_right(s_option_labels[i], 10, 0);

lv_obj_set_style_pad_top(s_option_labels[i], 2, 0);

lv_obj_set_style_pad_bottom(s_option_labels[i], 2, 0);

lv_obj_set_style_text_font(s_option_labels[i], UI_FONT_LARGE, 0);

lv_obj_set_style_text_color(s_option_labels[i], OPTION_TEXT_NORMAL_COLOR, 0);

}

lv_obj_t *btn_row = lv_obj_create(s_scroll);

lv_obj_set_width(btn_row, LV_PCT(100));

lv_obj_set_height(btn_row, 172);

lv_obj_set_flex_flow(btn_row, LV_FLEX_FLOW_ROW);

lv_obj_set_flex_align(btn_row,

LV_FLEX_ALIGN_START,

LV_FLEX_ALIGN_CENTER,

LV_FLEX_ALIGN_CENTER);

lv_obj_set_style_pad_left(btn_row, 8, 0);

lv_obj_set_style_pad_right(btn_row, 8, 0);

lv_obj_set_style_pad_top(btn_row, 6, 0);

lv_obj_set_style_pad_bottom(btn_row, 8, 0);

lv_obj_set_style_pad_gap(btn_row, 8, 0);

lv_obj_set_style_bg_opa(btn_row, LV_OPA_TRANSP, 0);

lv_obj_set_style_border_width(btn_row, 0, 0);

lv_obj_clear_flag(btn_row, LV_OBJ_FLAG_SCROLLABLE);

const lv_color_t border_col = lv_color_white();

const lv_color_t checked_bg = lv_palette_main(LV_PALETTE_GREEN);

for (int i = 0; i < QUIZ_OPTION_COUNT; i++)

{

s_option_btns[i] = lv_btn_create(btn_row);

lv_obj_set_flex_grow(s_option_btns[i], 1);

lv_obj_set_height(s_option_btns[i], 172);

lv_obj_add_flag(s_option_btns[i], LV_OBJ_FLAG_CHECKABLE);

lv_obj_set_style_border_width(s_option_btns[i], 2, LV_PART_MAIN);

lv_obj_set_style_border_color(s_option_btns[i], border_col, LV_PART_MAIN);

lv_obj_set_style_border_opa(s_option_btns[i], LV_OPA_100, LV_PART_MAIN);

lv_obj_set_style_bg_color(s_option_btns[i], checked_bg, LV_PART_MAIN | LV_STATE_CHECKED);

lv_obj_set_style_bg_opa(s_option_btns[i], LV_OPA_100, LV_PART_MAIN | LV_STATE_CHECKED);

lv_obj_set_style_text_color(s_option_btns[i], lv_color_white(), LV_PART_MAIN | LV_STATE_CHECKED);

lv_obj_add_event_cb(

s_option_btns[i],

quiz_handle_option,

LV_EVENT_CLICKED,

(void *)(intptr_t)i);

lv_obj_t *lbl = lv_label_create(s_option_btns[i]);

lv_label_set_text_fmt(lbl, "%c", 'A' + i);

lv_obj_center(lbl);

lv_obj_set_style_text_font(lbl, UI_FONT_LARGE, 0);

}

s_submit_btn = lv_btn_create(btn_row);

lv_obj_set_flex_grow(s_submit_btn, 1);

lv_obj_set_height(s_submit_btn, 172);

lv_obj_set_style_border_width(s_submit_btn, 2, LV_PART_MAIN);

lv_obj_set_style_border_color(s_submit_btn, border_col, LV_PART_MAIN);

lv_obj_set_style_border_opa(s_submit_btn, LV_OPA_100, LV_PART_MAIN);

lv_obj_add_event_cb(s_submit_btn, quiz_handle_submit, LV_EVENT_CLICKED, NULL);

lv_obj_t *submit_lbl = lv_label_create(s_submit_btn);

lv_label_set_text(submit_lbl, "Submit");

lv_obj_center(submit_lbl);

lv_obj_set_style_text_font(submit_lbl, UI_FONT_LARGE, 0);

}

static void quiz_build_result_screen(void)

{

if (s_result_screen)

{

lv_obj_clean(s_result_screen);

}

else

{

s_result_screen = lv_obj_create(NULL);

}

lv_obj_set_size(s_result_screen, 640, 172);

lv_obj_clear_flag(s_result_screen, LV_OBJ_FLAG_SCROLLABLE);

lv_obj_set_style_bg_color(s_result_screen, lv_color_white(), 0);

lv_obj_set_style_pad_all(s_result_screen, 0, 0);

s_result_scroll = lv_obj_create(s_result_screen);

lv_obj_set_size(s_result_scroll, 640, 172);

lv_obj_align(s_result_scroll, LV_ALIGN_TOP_MID, 0, 0);

lv_obj_set_scroll_dir(s_result_scroll, LV_DIR_VER);

lv_obj_set_scrollbar_mode(s_result_scroll, LV_SCROLLBAR_MODE_AUTO);

lv_obj_clear_flag(s_result_scroll, LV_OBJ_FLAG_SCROLL_ELASTIC);

lv_obj_set_flex_flow(s_result_scroll, LV_FLEX_FLOW_COLUMN);

lv_obj_set_style_pad_all(s_result_scroll, 8, 0);

lv_obj_set_style_pad_gap(s_result_scroll, 6, 0);

lv_obj_set_style_bg_opa(s_result_scroll, LV_OPA_TRANSP, 0);

lv_obj_set_style_border_width(s_result_scroll, 0, 0);

uint8_t local_correct = 0;

for (uint8_t i = 0; i < s_state.question_count; i++)

{

if (s_state.answers[i] < QUIZ_OPTION_COUNT &&

s_state.answers[i] == s_state.questions[i].correct_index)

{

local_correct++;

}

}

int show_score = (s_state.server_score >= 0) ? s_state.server_score : local_correct;

int show_total = (s_state.server_total > 0) ? s_state.server_total : s_state.question_count;

if (show_total <= 0) show_total = 1;

int wrong_cnt = show_total - show_score;

if (wrong_cnt < 0) wrong_cnt = 0;

int acc = (show_score * 100) / show_total;

lv_obj_t *title = lv_label_create(s_result_scroll);

lv_label_set_text(title, "Result");

lv_obj_set_style_text_font(title, UI_FONT_NORMAL, 0);

lv_obj_set_style_text_color(title, lv_color_hex(0x111111), 0);

lv_obj_t *sum_row = lv_obj_create(s_result_scroll);

lv_obj_clear_flag(sum_row, LV_OBJ_FLAG_SCROLLABLE);

lv_obj_set_width(sum_row, LV_PCT(100));

lv_obj_set_height(sum_row, LV_SIZE_CONTENT);

lv_obj_set_flex_flow(sum_row, LV_FLEX_FLOW_ROW);

lv_obj_set_flex_align(sum_row,

LV_FLEX_ALIGN_SPACE_BETWEEN,

LV_FLEX_ALIGN_CENTER,

LV_FLEX_ALIGN_CENTER);

lv_obj_set_style_bg_opa(sum_row, LV_OPA_TRANSP, 0);

lv_obj_set_style_pad_all(sum_row, 0, 0);

lv_obj_t *score_lbl = lv_label_create(sum_row);

lv_label_set_text_fmt(score_lbl, "Score: %d/%d", show_score, show_total);

lv_obj_set_style_text_font(score_lbl, UI_FONT_LARGE, 0);

lv_obj_set_style_text_color(score_lbl, lv_color_hex(0x111111), 0);

lv_obj_t *acc_lbl = lv_label_create(sum_row);

lv_label_set_text_fmt(acc_lbl, "Acc: %d%%", acc);

lv_obj_set_style_text_font(acc_lbl, UI_FONT_LARGE, 0);

lv_obj_set_style_text_color(acc_lbl, lv_color_hex(0x111111), 0);

lv_obj_t *time_lbl = lv_label_create(sum_row);

lv_label_set_text(time_lbl, "Time: --:--");

lv_obj_set_style_text_font(time_lbl, UI_FONT_LARGE, 0);

lv_obj_set_style_text_color(time_lbl, lv_color_hex(0x111111), 0);

lv_obj_t *cw_row = lv_obj_create(s_result_scroll);

lv_obj_clear_flag(cw_row, LV_OBJ_FLAG_SCROLLABLE);

lv_obj_set_width(cw_row, LV_PCT(100));

lv_obj_set_height(cw_row, LV_SIZE_CONTENT);

lv_obj_set_flex_flow(cw_row, LV_FLEX_FLOW_ROW);

lv_obj_set_flex_align(cw_row,

LV_FLEX_ALIGN_START,

LV_FLEX_ALIGN_CENTER,

LV_FLEX_ALIGN_CENTER);

lv_obj_set_style_bg_opa(cw_row, LV_OPA_TRANSP, 0);

lv_obj_set_style_pad_all(cw_row, 0, 0);

lv_obj_set_style_pad_gap(cw_row, 18, 0);

lv_obj_t *c_lbl = lv_label_create(cw_row);

lv_label_set_text_fmt(c_lbl, "Correct: %d", show_score);

lv_obj_set_style_text_font(c_lbl, UI_FONT_NORMAL, 0);

lv_obj_set_style_text_color(c_lbl, lv_color_hex(0x1a7f37), 0);

lv_obj_t *w_lbl = lv_label_create(cw_row);

lv_label_set_text_fmt(w_lbl, "Wrong: %d", wrong_cnt);

lv_obj_set_style_text_font(w_lbl, UI_FONT_NORMAL, 0);

lv_obj_set_style_text_color(w_lbl, lv_color_hex(0xb42318), 0);

lv_obj_t *divider = lv_obj_create(s_result_scroll);

lv_obj_clear_flag(divider, LV_OBJ_FLAG_SCROLLABLE);

lv_obj_set_width(divider, LV_PCT(100));

lv_obj_set_height(divider, 1);

lv_obj_set_style_bg_color(divider, lv_color_hex(0xE6E6E6), 0);

lv_obj_set_style_bg_opa(divider, LV_OPA_100, 0);

lv_obj_set_style_pad_all(divider, 0, 0);

lv_obj_t *wl_title = lv_label_create(s_result_scroll);

lv_label_set_text(wl_title, "Wrong list:");

lv_obj_set_style_text_font(wl_title, UI_FONT_NORMAL, 0);

lv_obj_set_style_text_color(wl_title, lv_color_hex(0x111111), 0);

bool has_wrong = false;

#define CREATE_WRONG_ROW(_qno, _your_c, _ans_c)                                      \

do                                                                              \

{                                                                               \

lv_obj_t *row = lv_obj_create(s_result_scroll);                             \

lv_obj_clear_flag(row, LV_OBJ_FLAG_SCROLLABLE);                             \

lv_obj_set_width(row, LV_PCT(100));                                         \

lv_obj_set_height(row, LV_SIZE_CONTENT);                                    \

\

lv_obj_set_style_pad_all(row, 8, 0);                                        \

lv_obj_set_style_radius(row, 10, 0);                                        \

lv_obj_set_style_bg_color(row, lv_color_hex(0xF6F7F9), 0);                  \

lv_obj_set_style_bg_opa(row, LV_OPA_100, 0);                                \

lv_obj_set_style_border_width(row, 1, 0);                                   \

lv_obj_set_style_border_color(row, lv_color_hex(0xE6E6E6), 0);              \

\

\

lv_obj_clear_flag(row, LV_OBJ_FLAG_CLICKABLE);                              \

\

lv_obj_set_flex_flow(row, LV_FLEX_FLOW_ROW);                                \

lv_obj_set_flex_align(row,                                                  \

LV_FLEX_ALIGN_SPACE_BETWEEN,                          \

LV_FLEX_ALIGN_CENTER,                                 \

LV_FLEX_ALIGN_CENTER);                                \

\

lv_obj_t *left = lv_label_create(row);                                      \

lv_label_set_text_fmt(left, "Q%d   Your:%c  Ans:%c", (_qno), (_your_c), (_ans_c)); \

lv_obj_set_style_text_font(left, UI_FONT_NORMAL, 0);                        \

lv_obj_set_style_text_color(left, lv_color_hex(0x111111), 0);               \

\

\

} while (0)

if (s_state.server_wrong_count > 0)

{

for (uint8_t i = 0; i < s_state.server_wrong_count; i++)

{

const quiz_wrong_item_t *wrong = &s_state.server_wrong[i];

int q_index = -1;

for (uint8_t k = 0; k < s_state.question_count; k++)

{

if (strncmp(s_state.questions[k].id, wrong->id, sizeof(s_state.questions[k].id)) == 0)

{

q_index = (int)k;

break;

}

}

if (q_index < 0) q_index = 0;

char your_c = (wrong->your >= 0 && wrong->your < QUIZ_OPTION_COUNT) ? ('A' + wrong->your) : '-';

char ans_c  = (wrong->correct >= 0 && wrong->correct < QUIZ_OPTION_COUNT) ? ('A' + wrong->correct) : '-';

CREATE_WRONG_ROW(q_index + 1, your_c, ans_c);

has_wrong = true;

}

}

else

{

for (uint8_t i = 0; i < s_state.question_count; i++)

{

int your = (s_state.answers[i] < QUIZ_OPTION_COUNT) ? (int)s_state.answers[i] : -1;

int corr = (int)s_state.questions[i].correct_index;

if (your == corr)

{

continue;

}

char your_c = (your >= 0 && your < QUIZ_OPTION_COUNT) ? ('A' + your) : '-';

char ans_c  = (corr >= 0 && corr < QUIZ_OPTION_COUNT) ? ('A' + corr) : '-';

CREATE_WRONG_ROW(i + 1, your_c, ans_c);

has_wrong = true;

}

}

#undef CREATE_WRONG_ROW

if (!has_wrong)

{

lv_obj_t *all_good = lv_label_create(s_result_scroll);

lv_label_set_text(all_good, "All answers correct!");

lv_obj_set_style_text_font(all_good, UI_FONT_NORMAL, 0);

lv_obj_set_style_text_color(all_good, lv_color_hex(0x1a7f37), 0);

}

lv_obj_t *btn_row = lv_obj_create(s_result_scroll);

lv_obj_clear_flag(btn_row, LV_OBJ_FLAG_SCROLLABLE);

lv_obj_set_width(btn_row, LV_PCT(100));

lv_obj_set_height(btn_row, LV_SIZE_CONTENT);

lv_obj_set_flex_flow(btn_row, LV_FLEX_FLOW_ROW);

lv_obj_set_flex_align(btn_row,

LV_FLEX_ALIGN_SPACE_BETWEEN,

LV_FLEX_ALIGN_CENTER,

LV_FLEX_ALIGN_CENTER);

lv_obj_set_style_bg_opa(btn_row, LV_OPA_TRANSP, 0);

lv_obj_set_style_pad_all(btn_row, 0, 0);

lv_obj_set_style_pad_gap(btn_row, 10, 0);

lv_obj_t *retry_btn = lv_btn_create(btn_row);

lv_obj_set_height(retry_btn, 44);

lv_obj_set_flex_grow(retry_btn, 1);

lv_obj_set_style_bg_color(retry_btn, lv_palette_main(LV_PALETTE_BLUE), 0);

lv_obj_set_style_radius(retry_btn, 10, 0);

lv_obj_set_style_border_width(retry_btn, 1, 0);

lv_obj_set_style_border_color(retry_btn, lv_color_hex(0x2f6ee2), 0);

lv_obj_add_event_cb(retry_btn, quiz_handle_start_test, LV_EVENT_CLICKED, NULL);

lv_obj_t *retry_lbl = lv_label_create(retry_btn);

lv_label_set_text(retry_lbl, "Retry");

lv_obj_set_style_text_font(retry_lbl, UI_FONT_NORMAL, 0);

lv_obj_set_style_text_color(retry_lbl, lv_color_white(), 0);

lv_obj_center(retry_lbl);

lv_obj_t *back_btn = lv_btn_create(btn_row);

lv_obj_set_height(back_btn, 44);

lv_obj_set_flex_grow(back_btn, 1);

lv_obj_set_style_bg_color(back_btn, lv_color_hex(0xEDEFF2), 0);

lv_obj_set_style_radius(back_btn, 10, 0);

lv_obj_set_style_border_width(back_btn, 1, 0);

lv_obj_set_style_border_color(back_btn, lv_color_hex(0xD0D5DD), 0);

lv_obj_add_event_cb(back_btn, quiz_back_to_home, LV_EVENT_CLICKED, NULL);

lv_obj_t *back_lbl = lv_label_create(back_btn);

lv_label_set_text(back_lbl, "Back");

lv_obj_set_style_text_font(back_lbl, UI_FONT_NORMAL, 0);

lv_obj_set_style_text_color(back_lbl, lv_color_hex(0x111111), 0);

lv_obj_center(back_lbl);

}

static void quiz_show_results_screen(void)

{

quiz_build_result_screen();

lv_scr_load(s_result_screen);

if (s_result_scroll)

{

lv_obj_scroll_to_y(s_result_scroll, 0, LV_ANIM_OFF);

}

}

static void quiz_handle_option(lv_event_t *e)

{

if (lv_event_get_code(e) != LV_EVENT_CLICKED)

{

return;

}

int index = (int)(intptr_t)lv_event_get_user_data(e);

if (index < 0 || index >= QUIZ_OPTION_COUNT)

{

return;

}

for (int i = 0; i < QUIZ_OPTION_COUNT; i++)

{

if (i == index)

{

lv_obj_add_state(s_option_btns[i], LV_STATE_CHECKED);

}

else

{

lv_obj_clear_state(s_option_btns[i], LV_STATE_CHECKED);

}

}

for (int i = 0; i < QUIZ_OPTION_COUNT; i++)

{

if (i == index)

{

lv_obj_set_style_text_color(

s_option_labels[i],

OPTION_TEXT_ACTIVE_COLOR,

0);

}

else

{

lv_obj_set_style_text_color(

s_option_labels[i],

OPTION_TEXT_NORMAL_COLOR,

0);

}

}

s_state.answers[s_state.current_question] = (uint8_t)index;

}

static void quiz_sanitize_text(char *dst, size_t dst_sz, const char *src)

{

if (!dst || dst_sz == 0) return;

size_t w = 0;

int last_space = 1;

for (size_t r = 0; src && src[r] != '\0' && w + 1 < dst_sz; r++)

{

char c = src[r];

if (c == '\r' || c == '\n' || c == '\t') c = ' ';

if (c == ' ')

{

if (last_space) continue;

last_space = 1;

}

else

{

last_space = 0;

}

dst[w++] = c;

}

while (w > 0 && dst[w - 1] == ' ') w--;

dst[w] = '\0';

}

static void quiz_load_question(uint8_t index)

{

if (index >= s_state.question_count)

{

return;

}

lv_label_set_text_fmt(

s_progress_label,

"%d / %d",

index + 1,

s_state.question_count

);

char stem_clean[QUIZ_TEXT_LEN + 1];

quiz_sanitize_text(stem_clean, sizeof(stem_clean), s_state.questions[index].stem);

char q_line[QUIZ_TEXT_LEN + 16];

snprintf(q_line, sizeof(q_line), "Q%d: %s", index + 1, stem_clean);

lv_label_set_text(s_question_label, q_line);

for (int i = 0; i < QUIZ_OPTION_COUNT; i++)

{

char opt_clean[QUIZ_TEXT_LEN + 1];

quiz_sanitize_text(opt_clean, sizeof(opt_clean), s_state.questions[index].options[i]);

lv_label_set_text_fmt(

s_option_labels[i],

"%c. %s",

'A' + i,

opt_clean

);

lv_obj_clear_state(s_option_btns[i], LV_STATE_CHECKED);

lv_obj_set_style_text_color(s_option_labels[i], OPTION_TEXT_NORMAL_COLOR, 0);

}

if (s_state.answers[index] < QUIZ_OPTION_COUNT)

{

uint8_t sel = s_state.answers[index];

lv_obj_add_state(s_option_btns[sel], LV_STATE_CHECKED);

lv_obj_set_style_text_color(s_option_labels[sel], OPTION_TEXT_ACTIVE_COLOR, 0);

}

lv_obj_scroll_to_y(s_scroll, 0, LV_ANIM_OFF);

}

static void quiz_finish_and_upload(void)

{

s_state.test_finished = true;

quiz_reset_server_result();

if (s_user_id[0] == '\0')

{

quiz_show_toast("User id missing", 1600);

return;

}

if (s_quiz_id[0] == '\0')

{

quiz_show_toast("Quiz id missing", 1600);

return;

}

uint8_t correct = 0;

s_state.server_wrong_count = 0;

for (uint8_t i = 0; i < s_state.question_count; i++)

{

int your_choice = (s_state.answers[i] < QUIZ_OPTION_COUNT) ? s_state.answers[i] : -1;

int correct_choice = s_state.questions[i].correct_index;

if (your_choice == correct_choice)

{

correct++;

}

else if (s_state.server_wrong_count < QUIZ_MAX_QUESTIONS)

{

quiz_wrong_item_t *dst = &s_state.server_wrong[s_state.server_wrong_count];

memset(dst, 0, sizeof(*dst));

strncpy(dst->id, s_state.questions[i].id, sizeof(dst->id) - 1);

dst->correct = correct_choice;

dst->your = your_choice;

s_state.server_wrong_count++;

}

}

s_state.server_score = correct;

s_state.server_total = s_state.question_count;

char payload[256];

snprintf(payload, sizeof(payload),

"{\"user_id\":\"%s\",\"quiz_id\":\"%s\",\"score\":%d}",

s_user_id, s_quiz_id, s_state.server_score);

esp_err_t err = quiz_http_post_results(payload);

if (err == ESP_OK)

{

quiz_show_toast("Results uploaded. View results from home.", 1800);

if (s_home_screen)

{

lv_scr_load(s_home_screen);

}

}

else

{

quiz_show_toast("Upload failed", 1800);

}

}

static void quiz_hide_toast_cb(lv_timer_t *t)

{

LV_UNUSED(t);

if (s_toast_label)

{

lv_obj_add_flag(s_toast_label, LV_OBJ_FLAG_HIDDEN);

}

}

static void quiz_back_to_home(lv_event_t *e)

{

LV_UNUSED(e);

if (s_home_screen)

{

lv_scr_load(s_home_screen);

}

}

static void quiz_handle_submit(lv_event_t *e)

{

LV_UNUSED(e);

uint8_t idx = s_state.current_question;

if (idx >= s_state.question_count)

{

return;

}

if (s_state.answers[idx] >= QUIZ_OPTION_COUNT)

{

quiz_show_toast("Select an option", 1600);

return;

}

if (idx + 1 >= s_state.question_count)

{

quiz_finish_and_upload();

}

else

{

s_state.current_question++;

s_state.answers[s_state.current_question] = 0xFF;

quiz_load_question(s_state.current_question);

}

}

void quiz_app_create_ui(void)

{

if (s_home_screen)

{

lv_scr_load(s_home_screen);

return;

}

memset(&s_state, 0, sizeof(s_state));

memset(s_state.answers, 0xFF, sizeof(s_state.answers));

quiz_reset_server_result();

quiz_create_home_screen();

quiz_build_test_screen();

lv_scr_load(s_home_screen);

}

void quiz_app_set_user_id(const char *user_id)

{

if (user_id)

{

strncpy(s_user_id, user_id, sizeof(s_user_id) - 1);

s_user_id[sizeof(s_user_id) - 1] = '\0';

}

else

{

s_user_id[0] = '\0';

}

s_quiz_id[0] = '\0';

}

static void quiz_reset_server_result(void)

{

s_state.server_score = -1;

s_state.server_total = s_state.question_count;

s_state.server_wrong_count = 0;

memset(s_state.server_wrong, 0, sizeof(s_state.server_wrong));

}

static const quiz_question_t *quiz_find_question_by_id(const char *id)

{

if (!id)

{

return NULL;

}

for (uint8_t i = 0; i < s_state.question_count; i++)

{

if (strncmp(s_state.questions[i].id, id, sizeof(s_state.questions[i].id)) == 0)

{

return &s_state.questions[i];

}

}

return NULL;

}

main/quiz_app.h

#ifndef QUIZ_APP_H

#define QUIZ_APP_H

#ifdef __cplusplus

extern "C" {

#endif

void quiz_app_create_ui(void);

void quiz_app_set_user_id(const char *user_id);

#ifdef __cplusplus

}

#endif

#endif

main/user_config.h

#ifndef USER_CONFIG_H

#define USER_CONFIG_H

#define LCD_HOST SPI3_HOST

#define Touch_SCL_NUM (GPIO_NUM_18)

#define Touch_SDA_NUM (GPIO_NUM_17)

#define ESP_SCL_NUM (GPIO_NUM_48)

#define ESP_SDA_NUM (GPIO_NUM_47)

#define EXAMPLE_PIN_NUM_LCD_CS     (GPIO_NUM_9)

#define EXAMPLE_PIN_NUM_LCD_PCLK   (GPIO_NUM_10)

#define EXAMPLE_PIN_NUM_LCD_DATA0  (GPIO_NUM_11)

#define EXAMPLE_PIN_NUM_LCD_DATA1  (GPIO_NUM_12)

#define EXAMPLE_PIN_NUM_LCD_DATA2  (GPIO_NUM_13)

#define EXAMPLE_PIN_NUM_LCD_DATA3  (GPIO_NUM_14)

#define EXAMPLE_PIN_NUM_LCD_RST    (GPIO_NUM_21)

#define EXAMPLE_PIN_NUM_BK_LIGHT   (GPIO_NUM_8)

#define EXAMPLE_PIN_NUM_TOUCH_ADDR        0x3b

#define EXAMPLE_PIN_NUM_TOUCH_RST         (-1)

#define EXAMPLE_PIN_NUM_TOUCH_INT         (-1)

#define EXAMPLE_LVGL_TICK_PERIOD_MS    5

#define EXAMPLE_LVGL_TASK_MAX_DELAY_MS 500

#define EXAMPLE_LVGL_TASK_MIN_DELAY_MS 5

#define EXAMPLE_RTC_ADDR 0x51

#define EXAMPLE_IMU_ADDR 0x6b

#define USER_DISP_ROT_90    1

#define USER_DISP_ROT_NONO  0

#define Rotated USER_DISP_ROT_90

#define Backlight_Testing 0

#if (Rotated == USER_DISP_ROT_NONO)

#define EXAMPLE_LCD_H_RES 172

#define EXAMPLE_LCD_V_RES 640

#else

#define EXAMPLE_LCD_H_RES 640

#define EXAMPLE_LCD_V_RES 172

#endif

#define LCD_NOROT_HRES     172

#define LCD_NOROT_VRES     640

#define LVGL_DMA_BUFF_LEN (LCD_NOROT_HRES * 64 * 2)

#define LVGL_SPIRAM_BUFF_LEN (EXAMPLE_LCD_H_RES * EXAMPLE_LCD_V_RES * 2)

#ifndef APP_WIFI_SSID

#define APP_WIFI_SSID "JD"

#endif

#ifndef APP_WIFI_PASS

#define APP_WIFI_PASS "123456789"

#endif

#ifndef APP_SERVER_HOST

#define APP_SERVER_HOST "10.57.246.86"

#endif

#ifndef APP_SERVER_PORT

#define APP_SERVER_PORT 8000

#endif

#ifndef APP_API_LOGIN_PATH

#define APP_API_LOGIN_PATH "/client/login"

#endif

#ifndef APP_API_STATUS_PATH

#define APP_API_STATUS_PATH "/auth/status"

#endif

#ifndef APP_API_QUESTIONS_PATH

#define APP_API_QUESTIONS_PATH "/questions"

#endif

#ifndef APP_API_SUBMIT_PATH

#define APP_API_SUBMIT_PATH "/submit"

#endif

#endif

components/button_bsp/button_bsp.c

#include "button_bsp.h"

#include "multi_button.h"

#include "esp_err.h"

#include "esp_log.h"

#include "esp_timer.h"

#include "driver/gpio.h"

EventGroupHandle_t boot_groups;

EventGroupHandle_t pwr_groups;

static Button button1;

#define USER_KEY_1 0

#define button1_id 1

#define button1_active 0

static Button button2;

#define USER_KEY_2 16

#define button2_id 2

#define button2_active 0

static void on_button2_press_repeat(Button* btn_handle);

static void on_button2_single_click(Button* btn_handle);

static void on_button2_double_click(Button* btn_handle);

static void on_button2_long_press_start(Button* btn_handle);

static void on_button2_long_press_hold(Button* btn_handle);

static void on_button2_press_down(Button* btn_handle);

static void on_button2_press_up(Button* btn_handle);

static void on_boot_single_click(Button* btn_handle);

static void on_boot_double_click(Button* btn_handle);

static void on_boot_long_press_start(Button* btn_handle);

static void on_boot_press_up(Button* btn_handle);

static void clock_task_callback(void *arg)

{

button_ticks();

}

static uint8_t read_button_GPIO(uint8_t button_id)

{

switch (button_id)

{

case button1_id:

return gpio_get_level(USER_KEY_1);

case button2_id:

return gpio_get_level(USER_KEY_2);

default:

break;

}

return 1;

}

static void gpio_init(void)

{

gpio_config_t gpio_conf = {};

gpio_conf.intr_type = GPIO_INTR_DISABLE;

gpio_conf.mode = GPIO_MODE_INPUT;

gpio_conf.pin_bit_mask = ((uint64_t)0x01<<USER_KEY_2) | ((uint64_t)0x01<<USER_KEY_1);

gpio_conf.pull_down_en = GPIO_PULLDOWN_DISABLE;

gpio_conf.pull_up_en = GPIO_PULLUP_ENABLE;

ESP_ERROR_CHECK_WITHOUT_ABORT(gpio_config(&gpio_conf));

}

void button_Init(void)

{

boot_groups = xEventGroupCreate();

pwr_groups = xEventGroupCreate();

gpio_init();

button_init(&button1, read_button_GPIO, button1_active , button1_id);

button_attach(&button1,BTN_SINGLE_CLICK,on_boot_single_click);

button_attach(&button1,BTN_LONG_PRESS_START,on_boot_long_press_start);

button_attach(&button1,BTN_PRESS_REPEAT,on_boot_double_click);

button_attach(&button1,BTN_PRESS_UP,on_boot_press_up);

button_init(&button2, read_button_GPIO, button2_active , button2_id);

button_attach(&button2,BTN_PRESS_REPEAT,on_button2_press_repeat);

button_attach(&button2,BTN_SINGLE_CLICK,on_button2_single_click);

button_attach(&button2,BTN_DOUBLE_CLICK,on_button2_double_click);

button_attach(&button2,BTN_LONG_PRESS_START,on_button2_long_press_start);

button_attach(&button2,BTN_PRESS_DOWN,on_button2_press_down);

button_attach(&button2,BTN_PRESS_UP,on_button2_press_up);

button_attach(&button2,BTN_LONG_PRESS_HOLD,on_button2_long_press_hold);

const esp_timer_create_args_t clock_tick_timer_args =

{

.callback = &clock_task_callback,

.name = "clock_task",

.arg = NULL,

};

esp_timer_handle_t clock_tick_timer = NULL;

ESP_ERROR_CHECK(esp_timer_create(&clock_tick_timer_args, &clock_tick_timer));

ESP_ERROR_CHECK(esp_timer_start_periodic(clock_tick_timer, 1000 * 5));

button_start(&button2);

button_start(&button1);

}

static void on_button2_press_repeat(Button* btn_handle)

{

xEventGroupSetBits(pwr_groups,set_bit_button(0));

}

static void on_button2_single_click(Button* btn_handle)

{

xEventGroupSetBits(pwr_groups,set_bit_button(0));

}

static void on_button2_double_click(Button* btn_handle)

{

}

static void on_button2_long_press_start(Button* btn_handle)

{

xEventGroupSetBits(pwr_groups,set_bit_button(1));

}

static void on_button2_long_press_hold(Button* btn_handle)

{

}

static void on_button2_press_down(Button* btn_handle)

{

}

static void on_button2_press_up(Button* btn_handle)

{

xEventGroupSetBits(pwr_groups,set_bit_button(2));

}

static void on_boot_single_click(Button* btn_handle)

{

xEventGroupSetBits(boot_groups,set_bit_button(0));

}

static void on_boot_double_click(Button* btn_handle)

{

xEventGroupSetBits(boot_groups,set_bit_button(1));

}

static void on_boot_long_press_start(Button* btn_handle)

{

xEventGroupSetBits(boot_groups,set_bit_button(2));

}

static void on_boot_press_up(Button* btn_handle)

{

xEventGroupSetBits(boot_groups,set_bit_button(3));

}

uint8_t user_button_get_repeat_count(void)

{

return (button_get_repeat_count(&button2));

}

uint8_t user_boot_get_repeat_count(void)

{

return (button_get_repeat_count(&button1));

}

components/button_bsp/button_bsp.h

#ifndef BUTTON_BSP_H

#define BUTTON_BSP_H

#include "freertos/FreeRTOS.h"

#include "freertos/task.h"

#include "freertos/event_groups.h"

#ifdef __cplusplus

extern "C" {

#endif

extern EventGroupHandle_t boot_groups;

extern EventGroupHandle_t pwr_groups;

#define set_bit_button(x) ((uint32_t)(0x01)<<(x))

#define get_bit_button(x,y) (((uint32_t)(x)>>(y)) & 0x01)

#define set_bit_all 0x00ffffff

#define set_bit_data(x,y) (x |= (0x01<<y))

#define clr_bit_data(x,y) (x &= ~(0x01<<y))

#define get_bit_data(x,y) ((x>>y) & 0x01)

#define rset_bit_data(x) ((uint32_t)0x01<<(x))

void button_Init(void);

uint8_t user_button_get_repeat_count(void);

uint8_t user_boot_get_repeat_count(void);

#ifdef __cplusplus

}

#endif

#endif

components/button_bsp/multi_button.c

#include "multi_button.h"

#define EVENT_CB(ev)   do { if(handle->cb[ev]) handle->cb[ev](handle); } while(0)

static Button* head_handle = NULL;

static void button_handler(Button* handle);

static inline uint8_t button_read_level(Button* handle);

void button_init(Button* handle, uint8_t(*pin_level)(uint8_t), uint8_t active_level, uint8_t button_id)

{

if (!handle || !pin_level) return;

memset(handle, 0, sizeof(Button));

handle->event = (uint8_t)BTN_NONE_PRESS;

handle->hal_button_level = pin_level;

handle->button_level = !active_level;

handle->active_level = active_level;

handle->button_id = button_id;

handle->state = BTN_STATE_IDLE;

}

void button_attach(Button* handle, ButtonEvent event, BtnCallback cb)

{

if (!handle || event >= BTN_EVENT_COUNT) return;

handle->cb[event] = cb;

}

void button_detach(Button* handle, ButtonEvent event)

{

if (!handle || event >= BTN_EVENT_COUNT) return;

handle->cb[event] = NULL;

}

ButtonEvent button_get_event(Button* handle)

{

if (!handle) return BTN_NONE_PRESS;

return (ButtonEvent)(handle->event);

}

uint8_t button_get_repeat_count(Button* handle)

{

if (!handle) return 0;

return handle->repeat;

}

void button_reset(Button* handle)

{

if (!handle) return;

handle->state = BTN_STATE_IDLE;

handle->ticks = 0;

handle->repeat = 0;

handle->event = (uint8_t)BTN_NONE_PRESS;

handle->debounce_cnt = 0;

}

int button_is_pressed(Button* handle)

{

if (!handle) return -1;

return (handle->button_level == handle->active_level) ? 1 : 0;

}

static inline uint8_t button_read_level(Button* handle)

{

return handle->hal_button_level(handle->button_id);

}

static void button_handler(Button* handle)

{

uint8_t read_gpio_level = button_read_level(handle);

if (handle->state > BTN_STATE_IDLE) {

handle->ticks++;

}

if (read_gpio_level != handle->button_level) {

if (++(handle->debounce_cnt) >= DEBOUNCE_TICKS) {

handle->button_level = read_gpio_level;

handle->debounce_cnt = 0;

}

} else {

handle->debounce_cnt = 0;

}

switch (handle->state) {

case BTN_STATE_IDLE:

if (handle->button_level == handle->active_level) {

handle->event = (uint8_t)BTN_PRESS_DOWN;

EVENT_CB(BTN_PRESS_DOWN);

handle->ticks = 0;

handle->repeat = 1;

handle->state = BTN_STATE_PRESS;

} else {

handle->event = (uint8_t)BTN_NONE_PRESS;

}

break;

case BTN_STATE_PRESS:

if (handle->button_level != handle->active_level) {

handle->event = (uint8_t)BTN_PRESS_UP;

EVENT_CB(BTN_PRESS_UP);

handle->ticks = 0;

handle->state = BTN_STATE_RELEASE;

} else if (handle->ticks > LONG_TICKS) {

handle->event = (uint8_t)BTN_LONG_PRESS_START;

EVENT_CB(BTN_LONG_PRESS_START);

handle->state = BTN_STATE_LONG_HOLD;

}

break;

case BTN_STATE_RELEASE:

if (handle->button_level == handle->active_level) {

handle->event = (uint8_t)BTN_PRESS_DOWN;

EVENT_CB(BTN_PRESS_DOWN);

if (handle->repeat < PRESS_REPEAT_MAX_NUM) {

handle->repeat++;

}

EVENT_CB(BTN_PRESS_REPEAT);

handle->ticks = 0;

handle->state = BTN_STATE_REPEAT;

} else if (handle->ticks > SHORT_TICKS) {

if (handle->repeat == 1) {

handle->event = (uint8_t)BTN_SINGLE_CLICK;

EVENT_CB(BTN_SINGLE_CLICK);

} else if (handle->repeat == 2) {

handle->event = (uint8_t)BTN_DOUBLE_CLICK;

EVENT_CB(BTN_DOUBLE_CLICK);

}

handle->state = BTN_STATE_IDLE;

}

break;

case BTN_STATE_REPEAT:

if (handle->button_level != handle->active_level) {

handle->event = (uint8_t)BTN_PRESS_UP;

EVENT_CB(BTN_PRESS_UP);

if (handle->ticks < SHORT_TICKS) {

handle->ticks = 0;

handle->state = BTN_STATE_RELEASE;

} else {

handle->state = BTN_STATE_IDLE;

}

} else if (handle->ticks > SHORT_TICKS) {

handle->state = BTN_STATE_PRESS;

}

break;

case BTN_STATE_LONG_HOLD:

if (handle->button_level == handle->active_level) {

handle->event = (uint8_t)BTN_LONG_PRESS_HOLD;

EVENT_CB(BTN_LONG_PRESS_HOLD);

} else {

handle->event = (uint8_t)BTN_PRESS_UP;

EVENT_CB(BTN_PRESS_UP);

handle->state = BTN_STATE_IDLE;

}

break;

default:

handle->state = BTN_STATE_IDLE;

break;

}

}

int button_start(Button* handle)

{

if (!handle) return -2;

Button* target = head_handle;

while (target) {

if (target == handle) return -1;

target = target->next;

}

handle->next = head_handle;

head_handle = handle;

return 0;

}

void button_stop(Button* handle)

{

if (!handle) return;

Button** curr;

for (curr = &head_handle; *curr; ) {

Button* entry = *curr;

if (entry == handle) {

*curr = entry->next;

entry->next = NULL;

return;

} else {

curr = &entry->next;

}

}

}

void button_ticks(void)

{

Button* target;

for (target = head_handle; target; target = target->next) {

button_handler(target);

}

}

components/button_bsp/multi_button.h

#ifndef _MULTI_BUTTON_H_

#define _MULTI_BUTTON_H_

#include <stdint.h>

#include <string.h>

#define TICKS_INTERVAL          5

#define DEBOUNCE_TICKS          3

#define SHORT_TICKS             (300 / TICKS_INTERVAL)

#define LONG_TICKS              (1000 / TICKS_INTERVAL)

#define PRESS_REPEAT_MAX_NUM    15

typedef struct _Button Button;

typedef void (*BtnCallback)(Button* btn_handle);

typedef enum {

BTN_PRESS_DOWN = 0,

BTN_PRESS_UP,

BTN_PRESS_REPEAT,

BTN_SINGLE_CLICK,

BTN_DOUBLE_CLICK,

BTN_LONG_PRESS_START,

BTN_LONG_PRESS_HOLD,

BTN_EVENT_COUNT,

BTN_NONE_PRESS

} ButtonEvent;

typedef enum {

BTN_STATE_IDLE = 0,

BTN_STATE_PRESS,

BTN_STATE_RELEASE,

BTN_STATE_REPEAT,

BTN_STATE_LONG_HOLD

} ButtonState;

struct _Button {

uint16_t ticks;

uint8_t  repeat : 4;

uint8_t  event : 4;

uint8_t  state : 3;

uint8_t  debounce_cnt : 3;

uint8_t  active_level : 1;

uint8_t  button_level : 1;

uint8_t  button_id;

uint8_t  (*hal_button_level)(uint8_t button_id);

BtnCallback cb[BTN_EVENT_COUNT];

Button* next;

};

#ifdef __cplusplus

extern "C" {

#endif

void button_init(Button* handle, uint8_t(*pin_level)(uint8_t), uint8_t active_level, uint8_t button_id);

void button_attach(Button* handle, ButtonEvent event, BtnCallback cb);

void button_detach(Button* handle, ButtonEvent event);

ButtonEvent button_get_event(Button* handle);

int  button_start(Button* handle);

void button_stop(Button* handle);

void button_ticks(void);

uint8_t button_get_repeat_count(Button* handle);

void button_reset(Button* handle);

int button_is_pressed(Button* handle);

#ifdef __cplusplus

}

#endif

#endif

components/i2c_bsp/i2c_bsp.c

#include <stdio.h>

#include "i2c_bsp.h"

#include "user_config.h"

#include "freertos/FreeRTOS.h"

static i2c_master_bus_handle_t user_i2c_port0_handle = NULL;

static i2c_master_bus_handle_t user_i2c_port1_handle = NULL;

i2c_master_dev_handle_t disp_touch_dev_handle = NULL;

i2c_master_dev_handle_t rtc_dev_handle = NULL;

i2c_master_dev_handle_t imu_dev_handle = NULL;

static uint32_t i2c_data_pdMS_TICKS = 0;

static uint32_t i2c_done_pdMS_TICKS = 0;

void i2c_master_Init(void)

{

i2c_data_pdMS_TICKS = pdMS_TO_TICKS(5000);

i2c_done_pdMS_TICKS = pdMS_TO_TICKS(1000);

i2c_master_bus_config_t i2c_bus_config =

{

.clk_source = I2C_CLK_SRC_DEFAULT,

.i2c_port = I2C_NUM_0,

.scl_io_num = ESP_SCL_NUM,

.sda_io_num = ESP_SDA_NUM,

.glitch_ignore_cnt = 7,

.flags = {

.enable_internal_pullup = true,

},

};

ESP_ERROR_CHECK(i2c_new_master_bus(&i2c_bus_config, &user_i2c_port0_handle));

i2c_bus_config.scl_io_num = Touch_SCL_NUM;

i2c_bus_config.sda_io_num = Touch_SDA_NUM;

i2c_bus_config.i2c_port = I2C_NUM_1;

ESP_ERROR_CHECK(i2c_new_master_bus(&i2c_bus_config, &user_i2c_port1_handle));

i2c_device_config_t dev_cfg =

{

.dev_addr_length = I2C_ADDR_BIT_LEN_7,

.scl_speed_hz = 300000,

};

dev_cfg.device_address = EXAMPLE_RTC_ADDR;

ESP_ERROR_CHECK(i2c_master_bus_add_device(user_i2c_port0_handle, &dev_cfg, &rtc_dev_handle));

dev_cfg.device_address = EXAMPLE_IMU_ADDR;

ESP_ERROR_CHECK(i2c_master_bus_add_device(user_i2c_port0_handle, &dev_cfg, &imu_dev_handle));

dev_cfg.device_address = EXAMPLE_PIN_NUM_TOUCH_ADDR;

ESP_ERROR_CHECK(i2c_master_bus_add_device(user_i2c_port1_handle, &dev_cfg, &disp_touch_dev_handle));

}

uint8_t i2c_writr_buff(i2c_master_dev_handle_t dev_handle,int reg,uint8_t *buf,uint8_t len)

{

uint8_t ret;

uint8_t *pbuf = NULL;

ret = i2c_master_bus_wait_all_done(user_i2c_port0_handle,i2c_done_pdMS_TICKS);

if(ret != ESP_OK)

return ret;

if(reg == -1)

{

ret = i2c_master_transmit(dev_handle,buf,len,i2c_data_pdMS_TICKS);

}

else

{

pbuf = (uint8_t*)malloc(len+1);

pbuf[0] = reg;

for(uint8_t i = 0; i<len; i++)

{

pbuf[i+1] = buf[i];

}

ret = i2c_master_transmit(dev_handle,pbuf,len+1,i2c_data_pdMS_TICKS);

free(pbuf);

pbuf = NULL;

}

return ret;

}

uint8_t i2c_master_write_read_dev(i2c_master_dev_handle_t dev_handle,uint8_t *writeBuf,uint8_t writeLen,uint8_t *readBuf,uint8_t readLen)

{

uint8_t ret;

ret = i2c_master_bus_wait_all_done(user_i2c_port0_handle,i2c_done_pdMS_TICKS);

if(ret != ESP_OK)

return ret;

ret = i2c_master_transmit_receive(dev_handle,writeBuf,writeLen,readBuf,readLen,i2c_data_pdMS_TICKS);

return ret;

}

uint8_t i2c_read_buff(i2c_master_dev_handle_t dev_handle,int reg,uint8_t *buf,uint8_t len)

{

uint8_t ret;

uint8_t addr = 0;

ret = i2c_master_bus_wait_all_done(user_i2c_port0_handle,i2c_done_pdMS_TICKS);

if(ret != ESP_OK)

return ret;

if( reg == -1 )

{ret = i2c_master_receive(dev_handle, buf,len, i2c_data_pdMS_TICKS);}

else

{addr = (uint8_t)reg; ret = i2c_master_transmit_receive(dev_handle,&addr,1,buf,len,i2c_data_pdMS_TICKS);}

return ret;

}

uint8_t i2c_master_touch_write_read(i2c_master_dev_handle_t dev_handle,uint8_t *writeBuf,uint8_t writeLen,uint8_t *readBuf,uint8_t readLen)

{

uint8_t ret;

ret = i2c_master_bus_wait_all_done(user_i2c_port1_handle,i2c_done_pdMS_TICKS);

if(ret != ESP_OK)

return ret;

ret = i2c_master_transmit_receive(dev_handle,writeBuf,writeLen,readBuf,readLen,i2c_data_pdMS_TICKS);

return ret;

}

components/i2c_bsp/i2c_bsp.h

#ifndef I2C_BSP_H

#define I2C_BSP_H

#include "driver/i2c_master.h"

extern i2c_master_dev_handle_t disp_touch_dev_handle;

extern i2c_master_dev_handle_t rtc_dev_handle;

extern i2c_master_dev_handle_t imu_dev_handle;

#ifdef __cplusplus

extern "C" {

#endif

void i2c_master_Init(void);

uint8_t i2c_writr_buff(i2c_master_dev_handle_t dev_handle,int reg,uint8_t *buf,uint8_t len);

uint8_t i2c_master_write_read_dev(i2c_master_dev_handle_t dev_handle,uint8_t *writeBuf,uint8_t writeLen,uint8_t *readBuf,uint8_t readLen);

uint8_t i2c_read_buff(i2c_master_dev_handle_t dev_handle,int reg,uint8_t *buf,uint8_t len);

uint8_t i2c_master_touch_write_read(i2c_master_dev_handle_t dev_handle,uint8_t *writeBuf,uint8_t writeLen,uint8_t *readBuf,uint8_t readLen);

#ifdef __cplusplus

}

#endif

#endif

components/lcd_bl_pwm_bsp/lcd_bl_pwm_bsp.c

#include <stdio.h>

#include "lcd_bl_pwm_bsp.h"

#include "esp_err.h"

#include "driver/ledc.h"

#include "driver/gpio.h"

#include "user_config.h"

void gpio_init(void)

{

gpio_config_t gpio_conf = {};

gpio_conf.intr_type = GPIO_INTR_DISABLE;

gpio_conf.mode = GPIO_MODE_OUTPUT;

gpio_conf.pin_bit_mask = ((uint64_t)0X01<<EXAMPLE_PIN_NUM_BK_LIGHT);

gpio_conf.pull_down_en = GPIO_PULLDOWN_DISABLE;

gpio_conf.pull_up_en = GPIO_PULLUP_ENABLE;

ESP_ERROR_CHECK_WITHOUT_ABORT(gpio_config(&gpio_conf));

}

void lcd_bl_pwm_bsp_init(uint16_t duty)

{

ledc_timer_config_t timer_conf =

{

.speed_mode =  LEDC_LOW_SPEED_MODE,

.duty_resolution = LEDC_TIMER_8_BIT,

.timer_num =  LEDC_TIMER_3,

.freq_hz = 50 * 1000,

.clk_cfg = LEDC_SLOW_CLK_RC_FAST,

};

ledc_channel_config_t ledc_conf =

{

.gpio_num = EXAMPLE_PIN_NUM_BK_LIGHT,

.speed_mode = LEDC_LOW_SPEED_MODE,

.channel =  LEDC_CHANNEL_1,

.intr_type =  LEDC_INTR_DISABLE,

.timer_sel = LEDC_TIMER_3,

.duty = duty,

.hpoint = 0,

};

ESP_ERROR_CHECK_WITHOUT_ABORT(ledc_timer_config(&timer_conf));

ESP_ERROR_CHECK_WITHOUT_ABORT(ledc_channel_config(&ledc_conf));

}

void setUpduty(uint16_t duty)

{

ESP_ERROR_CHECK_WITHOUT_ABORT(ledc_set_duty(LEDC_LOW_SPEED_MODE, LEDC_CHANNEL_1, duty));

ESP_ERROR_CHECK_WITHOUT_ABORT(ledc_update_duty(LEDC_LOW_SPEED_MODE, LEDC_CHANNEL_1));

}

components/lcd_bl_pwm_bsp/lcd_bl_pwm_bsp.h

#ifndef LCD_BL_PWM_BSP_H

#define LCD_BL_PWM_BSP_H

#define  LCD_PWM_MODE_0   (0xff-0)

#define  LCD_PWM_MODE_25  (0xff-25)

#define  LCD_PWM_MODE_50  (0xff-50)

#define  LCD_PWM_MODE_75  (0xff-75)

#define  LCD_PWM_MODE_100 (0xff-100)

#define  LCD_PWM_MODE_125 (0xff-125)

#define  LCD_PWM_MODE_150 (0xff-150)

#define  LCD_PWM_MODE_175 (0xff-175)

#define  LCD_PWM_MODE_200 (0xff-200)

#define  LCD_PWM_MODE_225 (0xff-225)

#define  LCD_PWM_MODE_255 (0xff-255)

#ifdef __cplusplus

extern "C" {

#endif

void lcd_bl_pwm_bsp_init(uint16_t duty);

void setUpduty(uint16_t duty);

#ifdef __cplusplus

}

#endif

#endif